| Photometric Test Report |  |  |
| --- | --- | --- |
| Report Number: POTS/GJ13065 | Report Date: 15-02-2013 | Prepared By: G John |
| Test Laboratory: Photometric and Optical Testing Services, Cotswold Business Centre, 42 A P Ellis Road, Rissington Business Park, Upper Rissington, Gloucestershire, GL54 2QB |  |  |
| Company Registration Number: Registered in England & Wales No. OC352911 |  |  |
| Registered Address: Thistle Down Barn, Holcot Lane, Sywell, Northampton, NN6 0BG |  |  |

| Client Details |  |
| --- | --- |
| Company: Astro Lighting | Email: technical@astrolighting.co.uk |
| Address: Astro Lighting Limited, G2 River Way, Harlow CM20 2DP, Great Britain |  |

| Details of Product Tested |  |
| --- | --- |
| Manufacturer: Astro Lighting | Source Type: Domestic light |
| Model: TRIMLESS 12 V HALOGEN LAMP | Serial Number: 5623 |
| Lamp Type: HALOGEN | Lamp serial number: 1562 |
| Power Supply Used: Uninterruptible AC power supply |  |
| Voltage(AC V) = 242.0 | Current (mA)= 204 |
| Power (Watts)= 48.96 | Power factor= 0.9904 |

| Integrating Sphere Test |  |  |
| --- | --- | --- |
| Date of Test: 05-02-2013 |  | Ambient Temperature:25°C |
| Measurement Filename: 5623 TRIMLESS 12 V HALOGEN LAMP |  |  |
| Instrument Used: Labsphere model CSLMS HALOGEN 4060 integrating sphere spectroradiometer |  |  |
| Integrating Sphere Size: 1m |  | Measurement Geometry (2 / 4): 2 |
| Sample Orientation: Horizontal |  | Auxiliary Correction Applied: YES |
| Comments: |  |  |
| Date of Last Calibration (Operating Hours): 08-11-2012 (02:36) |  | Spectral Flux Standard Lamp Used: SCL-1400 |
| Standard Lamp Serial Number: K75 |  | Traceable: to NIST standards |
| Calibration Certificate Number: DM-02008-001 |  | Calibration Certificate Date: 19th February 2010 |
| Calibration Lamp Uncertainty: ± 0.67% (k=2) |  |  |
| Results |  |  |
| Flux (lumens): 388.7 |  | Bare Lamp lumens: 503.6 |
| CIE 1931 Chromaticity Cx: 0.4422 |  | CIE 1931 Chromaticity Cy: 0.4097 |
| CRI (%):98.59 |  | CCT (K): 2956 |
| LOR(%): 77.2 |  |  |

Figure 1: Spectral Irradiance

Figure 2: CIE 1931 diagram.

| Goniophotometer Test |  |  |  |
| --- | --- | --- | --- |
| Date of Test: 05-02-2013 |  | Ambient Temperature: 25°C |  |
| Measurement Filename: 5623 TRIMLESS 12 V HALOGEN LAMP |  |  |  |
| Instrument Used: Radiant Imaging NFMS0800 Goniometer with ProMetric PM-1200N-1 Imaging Photometer |  |  |  |
| Photometer Working Distance: 1.5 m |  | Measurement Geometry: Near-Field |  |
| Comments: |  |  |  |
| Reference Photometer Used: Specbos1201 |  | Reference Photometer Serial Number: 2911670 |  |
| Traceable: to NPL standards, UKAS Accredited |  | Calibration Certificate Number: 121104 |  |
| Calibration Certificate Date: 25th October 2011 |  | Sample Stabilisation Time (minutes):75 |  |
| Reference Photometer Calibration Uncertainty: ± 2.4% (k=2, 20-200 lux, CIE illuminant A source) |  |  |  |
| Scan Set Up |  |  |  |
| Direction | Range |  | Increment |
| Inclination Zone 1 | 0-90° |  | 3° |
| Azimuth | 0-360° |  | 10° |
| Results |  |  |  |
| Integrated Luminous Flux (lumens):388.7 | Peak Intensity (3° Spot, candelas): 1230.6 |  | Efficacy (lumens/Watt): 7.94 |
| Beam Angle (50% of max intensity C0-180, degrees): 26.7 |  |  |  |
| Photometric Filename (IES LM-63-2002): 5623 TRIMLESS 12 V HALOGEN LAMP |  |  |  |
| IES File – Absolute or Relative Format? RELATIVE |  |  |  |
| Photometric Filename (EULUMDAT): 5623 TRIMLESS 12 V HALOGEN LAMP |  |  |  |
| EULUMDAT File – Absolute or Relative Format? RELATIVE |  |  |  |

Figure 3: Far-Field Luminous Intensity (C0-180, Cartesian Coordinates)

Figure 4: Far-Field Luminous Intensity (C0-180, C90-270, Polar Coordinates)

Figure 4. Illuminance cone diagram.

Table 1. UGR values

|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 |
| 3 | 1196.3 | 1199.9 | 1202.6 | 1205.4 | 1210.1 | 1216.0 | 1220.6 | 1222.2 | 1225.1 | 1225.3 | 1224.4 | 1220.1 | 1218.3 | 1210.5 | 1200.7 | 1194.2 | 1187.8 | 1182.8 | 1179.0 |
| 6 | 1130.9 | 1141.4 | 1137.0 | 1147.7 | 1158.0 | 1172.0 | 1185.3 | 1196.9 | 1212.3 | 1222.3 | 1230.6 | 1230.0 | 1227.4 | 1219.8 | 1207.0 | 1194.5 | 1181.0 | 1174.2 | 1168.1 |
| 9 | 946.2 | 959.6 | 963.8 | 969.4 | 990.0 | 1004.7 | 1016.8 | 1025.0 | 1055.6 | 1076.4 | 1097.7 | 1106.0 | 1124.7 | 1134.8 | 1134.1 | 1125.8 | 1121.8 | 1117.9 | 1112.6 |
| 12 | 679.8 | 679.0 | 684.9 | 707.3 | 700.7 | 715.6 | 750.6 | 754.7 | 774.5 | 798.3 | 817.9 | 842.4 | 880.6 | 883.3 | 903.5 | 936.0 | 935.1 | 944.8 | 955.3 |
| 15 | 431.7 | 432.2 | 431.5 | 461.1 | 451.9 | 464.5 | 499.3 | 492.9 | 510.0 | 522.4 | 535.5 | 544.0 | 590.4 | 586.2 | 608.4 | 659.3 | 655.6 | 682.5 | 699.8 |
| 18 | 263.1 | 262.1 | 272.3 | 271.9 | 286.2 | 298.9 | 310.7 | 331.0 | 333.3 | 340.0 | 340.5 | 349.6 | 343.9 | 355.7 | 366.4 | 377.3 | 408.9 | 418.0 | 437.9 |
| 21 | 167.6 | 166.2 | 171.3 | 178.0 | 185.7 | 197.2 | 210.6 | 221.2 | 226.0 | 231.7 | 229.6 | 229.5 | 227.7 | 225.4 | 226.7 | 232.2 | 239.1 | 245.9 | 259.3 |
| 24 | 120.7 | 119.1 | 120.9 | 125.9 | 131.8 | 137.6 | 143.4 | 149.2 | 148.5 | 154.6 | 155.6 | 160.2 | 160.0 | 160.6 | 160.1 | 159.0 | 158.3 | 161.2 | 169.3 |
| 27 | 99.1 | 98.3 | 99.0 | 99.5 | 100.5 | 102.3 | 104.9 | 105.4 | 101.1 | 102.8 | 105.1 | 111.0 | 114.0 | 114.7 | 116.5 | 118.7 | 120.1 | 121.3 | 126.8 |
| 30 | 79.0 | 79.1 | 77.3 | 79.2 | 78.9 | 80.7 | 82.3 | 79.8 | 77.8 | 76.2 | 76.6 | 78.6 | 84.2 | 85.1 | 88.3 | 92.4 | 93.2 | 97.2 | 101.8 |
| 33 | 53.3 | 52.2 | 52.4 | 52.6 | 55.4 | 57.6 | 57.0 | 58.7 | 58.1 | 57.4 | 57.2 | 59.2 | 60.7 | 63.2 | 65.1 | 66.3 | 70.9 | 74.1 | 79.2 |
| 36 | 32.1 | 30.4 | 32.5 | 34.2 | 35.5 | 37.4 | 38.1 | 39.9 | 39.3 | 39.7 | 40.3 | 42.1 | 42.5 | 43.2 | 43.5 | 44.5 | 48.6 | 50.5 | 55.2 |
| 39 | 20.9 | 20.2 | 21.0 | 22.8 | 23.6 | 25.0 | 25.6 | 26.2 | 27.2 | 26.7 | 28.2 | 28.1 | 28.6 | 28.5 | 28.8 | 28.9 | 30.4 | 33.4 | 35.0 |
| 42 | 16.0 | 15.8 | 16.4 | 16.7 | 16.9 | 17.9 | 18.4 | 18.5 | 19.0 | 18.8 | 19.7 | 20.1 | 20.1 | 19.6 | 19.8 | 20.0 | 21.0 | 22.0 | 22.7 |
| 45 | 14.0 | 14.3 | 14.2 | 14.2 | 14.5 | 15.3 | 15.3 | 15.0 | 15.1 | 15.1 | 15.6 | 15.8 | 15.8 | 15.3 | 15.2 | 14.9 | 15.2 | 15.9 | 16.6 |
| 48 | 11.8 | 12.3 | 12.1 | 12.1 | 12.9 | 14.0 | 14.1 | 13.5 | 13.2 | 13.7 | 13.8 | 13.6 | 13.3 | 12.8 | 12.5 | 11.8 | 11.8 | 12.5 | 13.3 |
| 51 | 8.0 | 8.6 | 8.5 | 8.5 | 9.2 | 10.8 | 11.3 | 11.3 | 10.4 | 11.5 | 11.3 | 11.0 | 10.6 | 10.5 | 10.1 | 9.9 | 9.8 | 10.2 | 10.7 |
| 54 | 3.9 | 5.1 | 4.4 | 4.7 | 5.3 | 7.0 | 6.8 | 7.3 | 6.3 | 7.5 | 7.5 | 6.9 | 6.8 | 7.7 | 7.5 | 7.7 | 7.7 | 7.9 | 8.1 |
| 57 | 0.5 | 1.0 | 2.0 | 2.4 | 1.2 | 3.9 | 3.4 | 4.3 | 3.2 | 3.8 | 4.3 | 3.8 | 3.8 | 4.9 | 4.9 | 5.1 | 5.0 | 5.4 | 5.3 |
| 60 | 0.0 | 0.0 | 0.6 | 0.7 | 1.0 | 2.4 | 2.1 | 2.4 | 1.1 | 1.5 | 2.7 | 2.3 | 2.3 | 3.5 | 3.6 | 3.0 | 3.1 | 3.2 | 3.2 |
| 63 | 0.0 | 0.0 | 0.0 | 0.5 | 1.3 | 1.8 | 1.8 | 0.6 | 0.6 | 0.5 | 2.2 | 1.8 | 1.8 | 3.0 | 3.0 | 1.9 | 0.9 | 2.0 | 1.7 |
| 66 | 0.0 | 0.0 | 1.0 | 0.5 | 1.3 | 1.4 | 1.7 | 0.0 | 0.5 | 0.2 | 1.0 | 1.6 | 1.6 | 2.6 | 2.7 | 1.6 | 1.0 | 0.9 | 1.7 |
| 69 | 0.0 | 0.0 | 0.6 | 1.0 | 0.8 | 0.8 | 1.6 | 0.4 | 0.9 | 0.8 | 1.8 | 1.5 | 1.5 | 2.4 | 2.5 | 1.4 | 1.3 | 1.5 | 1.3 |
| 72 | 0.2 | 0.0 | 0.0 | 0.0 | 0.9 | 0.6 | 1.5 | 0.8 | 0.0 | 0.4 | 1.7 | 0.5 | 1.5 | 2.2 | 2.3 | 0.7 | 0.9 | 1.4 | 0.9 |
| 75 | 0.0 | 0.0 | 0.0 | 0.5 | 0.4 | 0.5 | 1.4 | 1.1 | 0.0 | 0.7 | 1.7 | 0.0 | 0.7 | 2.1 | 1.4 | 1.3 | 0.6 | 0.7 | 1.1 |
| 78 | 0.0 | 0.0 | 0.2 | 0.5 | 0.8 | 0.0 | 0.6 | 0.8 | 0.3 | 0.2 | 1.6 | 0.7 | 0.7 | 1.9 | 1.9 | 1.3 | 1.0 | 1.4 | 1.4 |
| 81 | 0.0 | 0.0 | 0.0 | 0.4 | 0.0 | 0.5 | 0.0 | 0.0 | 0.0 | 0.2 | 1.0 | 1.0 | 0.7 | 1.7 | 1.8 | 0.7 | 0.6 | 0.7 | 0.8 |
| 84 | 0.0 | 0.0 | 0.0 | 0.4 | 0.0 | 0.0 | 0.0 | 0.5 | 0.2 | 0.0 | 1.5 | 1.4 | 1.4 | 0.8 | 0.9 | 1.3 | 0.9 | 1.4 | 1.1 |
| 87 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.3 | 0.5 | 0.2 | 0.0 | 0.0 | 1.0 | 1.3 | 0.0 | 1.0 | 1.0 | 1.3 | 1.3 | 0.9 | 0.6 |
| 90 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.5 | 0.4 | 0.6 | 0.8 | 0.5 | 0.7 | 0.0 | 0.4 | 0.3 |

Table 2a. Luminous intensity values, azimuth 0-180°

|  | 190 | 200 | 210 | 220 | 12 | 240 | 250 | 260 | 270 | 280 | 290 | 300 | 310 | 320 | 330 | 340 | 350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 | 1191.5 |
| 3 | 1177.6 | 1177.8 | 1179.8 | 1183.2 | 1189.1 | 1194.7 | 1195.9 | 1199.2 | 1200.5 | 1200.8 | 1199.2 | 1198.7 | 1196.7 | 1194.8 | 1193.2 | 1194.2 | 1195.6 |
| 6 | 1169.3 | 1173.2 | 1184.2 | 1196.5 | 1210.8 | 1220.2 | 1224.4 | 1223.7 | 1215.1 | 1201.9 | 1185.6 | 1172.1 | 1157.8 | 1144.4 | 1136.0 | 1128.8 | 1136.9 |
| 9 | 1120.7 | 1129.7 | 1140.9 | 1158.4 | 1169.8 | 1172.8 | 1163.9 | 1154.2 | 1127.3 | 1094.9 | 1053.3 | 1032.2 | 1009.5 | 988.9 | 965.2 | 959.0 | 956.7 |
| 12 | 961.5 | 969.4 | 986.7 | 976.7 | 978.1 | 989.8 | 964.2 | 940.3 | 908.5 | 864.1 | 819.8 | 793.8 | 742.6 | 714.2 | 712.3 | 686.5 | 679.3 |
| 15 | 710.4 | 708.0 | 735.2 | 706.2 | 703.0 | 723.0 | 683.8 | 673.1 | 644.4 | 605.5 | 561.7 | 544.5 | 490.9 | 466.6 | 469.1 | 436.0 | 434.7 |
| 18 | 444.7 | 459.2 | 445.2 | 449.6 | 451.2 | 448.9 | 460.0 | 444.2 | 429.2 | 399.8 | 377.0 | 338.5 | 314.4 | 295.1 | 277.9 | 276.6 | 264.3 |
| 21 | 263.1 | 271.8 | 278.2 | 282.4 | 289.9 | 299.3 | 304.5 | 302.7 | 297.2 | 278.0 | 258.6 | 237.3 | 216.5 | 199.3 | 188.2 | 178.2 | 169.2 |
| 24 | 172.6 | 179.8 | 191.6 | 202.9 | 212.5 | 220.3 | 227.7 | 227.9 | 226.0 | 212.1 | 198.6 | 181.8 | 167.6 | 153.5 | 141.4 | 130.5 | 123.1 |
| 27 | 131.3 | 139.4 | 147.5 | 155.7 | 165.9 | 175.5 | 181.6 | 179.4 | 175.5 | 165.1 | 155.0 | 142.8 | 132.0 | 122.4 | 115.3 | 108.5 | 101.5 |
| 30 | 106.5 | 110.0 | 118.4 | 124.2 | 131.6 | 138.6 | 136.9 | 132.5 | 125.8 | 119.8 | 110.8 | 107.4 | 100.8 | 96.2 | 93.5 | 87.0 | 82.2 |
| 33 | 82.2 | 84.7 | 88.1 | 95.8 | 99.5 | 97.5 | 96.2 | 85.1 | 81.3 | 77.9 | 74.0 | 69.5 | 68.7 | 65.9 | 62.5 | 61.8 | 56.2 |
| 36 | 56.6 | 58.6 | 60.7 | 66.0 | 67.3 | 65.0 | 62.1 | 49.7 | 49.2 | 47.5 | 46.4 | 43.4 | 41.2 | 38.9 | 38.3 | 38.7 | 34.3 |
| 39 | 36.9 | 35.7 | 37.5 | 41.3 | 41.7 | 40.1 | 36.9 | 31.1 | 30.1 | 30.3 | 28.5 | 27.4 | 25.5 | 24.3 | 24.1 | 24.0 | 22.9 |
| 42 | 23.6 | 23.7 | 23.8 | 25.1 | 25.5 | 25.3 | 24.0 | 21.6 | 20.8 | 20.6 | 19.9 | 19.0 | 17.4 | 16.9 | 17.6 | 17.9 | 17.0 |
| 45 | 17.1 | 17.4 | 17.5 | 17.9 | 18.4 | 18.5 | 18.1 | 17.5 | 17.0 | 16.4 | 15.8 | 14.9 | 13.8 | 13.8 | 15.0 | 15.0 | 14.3 |
| 48 | 13.9 | 14.2 | 14.2 | 14.6 | 15.3 | 15.7 | 15.9 | 15.3 | 15.6 | 14.5 | 13.7 | 12.7 | 12.6 | 12.2 | 13.5 | 12.9 | 11.9 |
| 51 | 11.4 | 12.1 | 12.3 | 12.5 | 13.2 | 13.6 | 14.3 | 12.6 | 13.7 | 11.8 | 11.7 | 10.3 | 11.2 | 9.8 | 10.4 | 9.6 | 8.0 |
| 54 | 8.5 | 9.3 | 10.0 | 10.1 | 10.8 | 10.6 | 10.8 | 8.4 | 9.8 | 7.5 | 8.1 | 6.5 | 7.8 | 6.4 | 6.0 | 5.3 | 4.7 |
| 57 | 5.5 | 6.3 | 7.2 | 6.7 | 7.2 | 7.4 | 7.0 | 4.8 | 5.7 | 4.0 | 5.0 | 3.4 | 4.1 | 3.2 | 2.8 | 2.8 | 0.8 |
| 60 | 3.3 | 4.5 | 4.9 | 4.4 | 4.4 | 5.1 | 4.5 | 2.9 | 2.5 | 1.6 | 3.1 | 0.8 | 2.5 | 2.0 | 1.5 | 1.8 | 0.3 |
| 63 | 2.3 | 3.4 | 3.6 | 3.1 | 3.1 | 3.8 | 3.4 | 2.2 | 2.1 | 1.9 | 2.3 | 0.5 | 1.8 | 1.6 | 0.0 | 0.0 | 0.0 |
| 66 | 1.0 | 2.9 | 3.1 | 2.6 | 2.6 | 3.2 | 2.9 | 1.9 | 1.7 | 1.7 | 1.9 | 0.0 | 1.6 | 0.9 | 0.0 | 0.0 | 0.0 |
| 69 | 1.9 | 2.6 | 2.8 | 2.3 | 2.3 | 2.8 | 1.7 | 1.8 | 1.3 | 0.8 | 1.7 | 0.0 | 1.4 | 0.9 | 0.0 | 0.0 | 0.0 |
| 72 | 0.9 | 2.3 | 2.5 | 2.1 | 2.0 | 2.5 | 1.5 | 1.7 | 0.6 | 1.5 | 1.5 | 0.0 | 1.3 | 0.9 | 0.0 | 0.0 | 0.0 |
| 75 | 1.6 | 2.0 | 2.2 | 1.3 | 1.8 | 2.3 | 2.0 | 1.6 | 0.9 | 0.5 | 0.7 | 0.0 | 1.2 | 0.8 | 0.0 | 0.0 | 0.0 |
| 78 | 1.5 | 1.4 | 2.0 | 1.4 | 1.7 | 2.0 | 0.9 | 1.0 | 0.8 | 0.9 | 0.6 | 0.0 | 1.1 | 0.8 | 0.0 | 0.0 | 0.4 |
| 81 | 1.5 | 1.7 | 1.8 | 0.9 | 1.7 | 1.8 | 1.3 | 1.0 | 0.8 | 1.0 | 0.8 | 0.0 | 1.0 | 0.0 | 0.0 | 0.2 | 0.0 |
| 84 | 1.1 | 1.7 | 1.6 | 1.5 | 1.6 | 1.7 | 1.6 | 1.4 | 1.1 | 1.4 | 0.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 87 | 0.9 | 1.5 | 1.5 | 1.4 | 1.5 | 1.5 | 1.1 | 1.4 | 0.8 | 0.9 | 0.3 | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 | 0.0 |
| 90 | 0.4 | 0.5 | 0.7 | 0.5 | 0.7 | 0.7 | 0.5 | 0.5 | 0.0 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 2b. Luminous intensity values, azimuth 190-350°

Signature:

_________________________________

Print Name:

GH JOHN

_________________________________

Date:

15-02-2013

_________________________________

Partner / Director
Duly authorised to sign on behalf of:

Photometric and Optical Testing Services LLP