| Photometric Test Report |  |  |
| --- | --- | --- |
| Report Number: POTS/GJ13076 | Report Date: 26-02-2013 | Prepared By: G John |
| Test Laboratory: Photometric and Optical Testing Services, Cotswold Business Centre, 42 A P Ellis Road, Rissington Business Park, Upper Rissington, Gloucestershire, GL54 2QB |  |  |
| Company Registration Number: Registered in England & Wales No. OC352911 |  |  |
| Registered Address: Thistle Down Barn, Holcot Lane, Sywell, Northampton, NN6 0BG |  |  |

| Client Details |  |
| --- | --- |
| Company: Astro Lighting | Email: technical@astrolighting.co.uk |
| Address: Astro Lighting Limited, G2 River Way, Harlow CM20 2DP, Great Britain |  |

| Details of Product Tested |  |
| --- | --- |
| Manufacturer: Astro Lighting | Source Type: Domestic light |
| Model: IXTRA SHAVER | Serial Number: 0598 |
| Lamp Type: 24w 2G11 PL-L compact fluorescent lamp |  |
| Power Supply Used: Uninterruptible AC power supply |  |
| Voltage(AC V) = 243.4 | Current (mA)= 132 |
| Power (Watts)= 20.54 | Power factor= 0.6398 |

| Integrating Sphere Test |  |  |
| --- | --- | --- |
| Date of Test: 20-12-2012 |  | Ambient Temperature:25°C |
| Measurement Filename: 0598 IXTRA SHAVER |  |  |
| Instrument Used: Labsphere model CSLMS HALOGEN 4060 integrating sphere spectroradiometer |  |  |
| Integrating Sphere Size: 1m |  | Measurement Geometry (2 / 4): 2 |
| Sample Orientation: Horizontal |  | Auxiliary Correction Applied: YES |
| Comments: |  |  |
| Date of Last Calibration (Operating Hours): 08-11-2012 (02:36) |  | Spectral Flux Standard Lamp Used: SCL-1400 |
| Standard Lamp Serial Number: K75 |  | Traceable: to NIST standards |
| Calibration Certificate Number: DM-02008-001 |  | Calibration Certificate Date: 19th February 2010 |
| Calibration Lamp Uncertainty: ± 0.67% (k=2) |  |  |
| Results |  |  |
| Flux (lumens): 865.5 |  | Rated lamp flux (lumens): 1800 |
| CIE 1931 Chromaticity Cx: 0.4497 |  | CIE 1931 Chromaticity Cy: 0.4135 |
| CRI (%):85.49 |  | CCT (K): 2868 |
| LOR (%): 48.1 |  |  |

Figure 1: Spectral Irradiance

Figure 2: CIE 1931 diagram.

| Goniophotometer Test |  |  |  |
| --- | --- | --- | --- |
| Date of Test: 20-12-2012 |  | Ambient Temperature: 25°C |  |
| Measurement Filename: 0598 IXTRA SHAVER |  |  |  |
| Instrument Used: Radiant Imaging NFMS0800 Goniometer with ProMetric PM-1200N-1 Imaging Photometer |  |  |  |
| Photometer Working Distance: 1.5 m |  | Measurement Geometry: Near-Field |  |
| Comments: |  |  |  |
| Reference Photometer Used: Specbos1201 |  | Reference Photometer Serial Number: 2911670 |  |
| Traceable: to NPL standards, UKAS Accredited |  | Calibration Certificate Number: 121104 |  |
| Calibration Certificate Date: 25th October 2011 |  | Sample Stabilisation Time (minutes):10 |  |
| Reference Photometer Calibration Uncertainty: ± 2.4% (k=2, 20-200 lux, CIE illuminant A source) |  |  |  |
| Scan Set Up |  |  |  |
| Direction | Range |  | Increment |
| Inclination Zone 1 | 0-90° |  | 3° |
| Azimuth | 0-360° |  | 10° |
| Results |  |  |  |
| Integrated Luminous Flux (lumens):865.5 | Peak Intensity (3° Spot, candelas): 157.2 |  | Efficacy (lumens/Watt): 42.14 |
| Beam Angle (50% of max intensity C0-180, degrees): 126.3 |  |  |  |
| Photometric Filename (IES LM-63-2002): 0598 IXTRA SHAVER |  |  |  |
| IES File – Absolute or Relative Format? Relative |  |  |  |
| Photometric Filename (EULUMDAT): 0598 IXTRA SHAVER |  |  |  |
| EULUMDAT File – Absolute or Relative Format? Relative |  |  |  |

Figure 3: Far-Field Luminous Intensity (C0-180, Cartesian Coordinates)

Figure 4: Far-Field Luminous Intensity (C0-180, C90-270, Polar Coordinates)

Figure 4. Illuminance cone diagram.

Table 1. UGR values

|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 |
| 5 | 143.6 | 143.3 | 143.0 | 142.1 | 142.3 | 143.7 | 144.9 | 144.9 | 144.2 | 144.4 | 145.6 | 147.1 | 147.3 | 146.5 | 145.8 | 145.7 | 145.3 | 144.5 | 143.9 |
| 10 | 142.0 | 143.0 | 144.3 | 143.3 | 143.0 | 143.8 | 145.6 | 145.8 | 146.4 | 149.2 | 147.6 | 146.2 | 146.1 | 144.9 | 147.6 | 149.4 | 144.7 | 140.8 | 142.0 |
| 15 | 144.2 | 145.7 | 141.4 | 139.9 | 145.0 | 144.8 | 145.4 | 148.9 | 150.9 | 147.1 | 147.1 | 148.4 | 148.3 | 147.6 | 141.4 | 143.0 | 144.9 | 140.2 | 144.1 |
| 20 | 142.5 | 145.5 | 139.0 | 143.9 | 144.9 | 147.8 | 146.8 | 150.0 | 148.8 | 153.5 | 150.7 | 148.6 | 146.5 | 147.6 | 142.6 | 142.8 | 146.5 | 138.0 | 142.7 |
| 25 | 137.0 | 141.4 | 139.9 | 144.1 | 144.3 | 149.1 | 147.0 | 148.8 | 149.1 | 145.9 | 151.4 | 151.4 | 152.3 | 145.8 | 146.6 | 143.0 | 143.2 | 135.1 | 136.9 |
| 30 | 135.1 | 140.4 | 142.4 | 136.6 | 148.2 | 149.3 | 150.5 | 151.5 | 152.9 | 150.4 | 152.3 | 152.3 | 151.6 | 147.5 | 143.3 | 143.0 | 141.9 | 135.2 | 139.0 |
| 35 | 134.5 | 134.8 | 137.0 | 141.3 | 144.5 | 149.7 | 150.6 | 148.5 | 153.2 | 155.0 | 151.6 | 151.9 | 152.2 | 149.1 | 143.9 | 143.7 | 139.4 | 130.0 | 138.5 |
| 40 | 132.5 | 130.7 | 137.7 | 139.4 | 144.5 | 146.4 | 152.6 | 152.0 | 157.2 | 154.5 | 149.2 | 152.1 | 148.3 | 149.4 | 142.3 | 143.8 | 132.1 | 130.8 | 134.1 |
| 45 | 126.9 | 127.8 | 135.2 | 137.0 | 142.6 | 148.7 | 145.6 | 154.0 | 154.0 | 151.8 | 155.3 | 152.4 | 151.4 | 145.7 | 142.3 | 140.2 | 132.7 | 129.6 | 129.7 |
| 50 | 123.7 | 125.6 | 129.4 | 136.3 | 141.4 | 140.4 | 151.1 | 149.6 | 157.0 | 154.2 | 152.3 | 146.1 | 149.6 | 143.5 | 139.2 | 136.5 | 122.8 | 126.0 | 127.0 |
| 55 | 115.7 | 120.9 | 128.9 | 130.9 | 136.6 | 147.3 | 145.3 | 150.4 | 149.0 | 153.6 | 150.7 | 154.4 | 149.3 | 142.7 | 141.8 | 130.9 | 124.1 | 122.4 | 128.1 |
| 60 | 116.9 | 120.0 | 121.7 | 131.7 | 139.7 | 144.9 | 142.9 | 146.8 | 150.1 | 148.9 | 149.4 | 148.0 | 145.2 | 139.1 | 137.8 | 130.3 | 121.7 | 122.2 | 118.8 |
| 65 | 107.7 | 112.4 | 120.9 | 127.2 | 129.7 | 138.8 | 140.5 | 146.5 | 148.6 | 146.8 | 149.0 | 148.3 | 140.9 | 135.3 | 132.5 | 122.0 | 114.7 | 115.6 | 115.0 |
| 70 | 105.9 | 112.5 | 116.4 | 124.0 | 130.5 | 137.6 | 140.4 | 144.5 | 145.9 | 146.6 | 145.8 | 146.1 | 139.0 | 138.9 | 131.3 | 119.2 | 109.8 | 109.1 | 108.9 |
| 75 | 99.0 | 102.7 | 108.4 | 119.3 | 126.0 | 129.2 | 137.2 | 143.0 | 144.1 | 143.7 | 142.6 | 140.0 | 137.6 | 133.1 | 124.2 | 113.3 | 103.8 | 104.8 | 102.7 |
| 80 | 93.8 | 97.1 | 104.9 | 113.8 | 122.1 | 127.6 | 133.2 | 139.5 | 139.8 | 142.4 | 136.7 | 136.6 | 136.9 | 132.6 | 117.8 | 109.1 | 103.1 | 100.1 | 98.6 |
| 85 | 86.6 | 92.5 | 96.3 | 106.1 | 114.3 | 122.3 | 125.7 | 134.1 | 131.1 | 138.4 | 135.7 | 134.1 | 128.6 | 123.1 | 114.0 | 107.0 | 96.9 | 93.8 | 93.9 |
| 90 | 80.4 | 86.5 | 92.1 | 102.7 | 112.3 | 119.2 | 127.1 | 131.6 | 127.3 | 134.6 | 131.8 | 133.0 | 126.1 | 120.2 | 104.5 | 101.9 | 93.4 | 87.1 | 85.3 |
| 95 | 73.1 | 76.1 | 84.9 | 92.0 | 104.1 | 117.0 | 120.3 | 127.5 | 128.5 | 125.5 | 130.1 | 128.0 | 120.7 | 111.3 | 104.4 | 91.6 | 88.1 | 83.0 | 78.6 |
| 100 | 67.3 | 70.8 | 83.4 | 92.8 | 103.1 | 107.1 | 115.4 | 119.5 | 117.1 | 121.4 | 124.1 | 124.0 | 113.2 | 107.8 | 95.9 | 90.6 | 83.5 | 74.9 | 72.5 |
| 105 | 62.8 | 64.9 | 77.3 | 86.3 | 92.5 | 102.4 | 108.8 | 116.9 | 115.6 | 118.3 | 118.6 | 115.2 | 111.1 | 101.2 | 92.6 | 84.0 | 77.7 | 67.8 | 61.8 |
| 110 | 52.7 | 59.7 | 67.4 | 78.4 | 88.2 | 95.7 | 106.7 | 111.4 | 113.3 | 112.4 | 117.7 | 110.5 | 103.6 | 91.5 | 85.7 | 82.3 | 67.8 | 58.6 | 57.1 |
| 115 | 42.6 | 48.6 | 58.8 | 71.4 | 81.9 | 91.9 | 98.9 | 102.0 | 105.5 | 108.2 | 110.6 | 104.5 | 98.0 | 91.5 | 78.6 | 73.6 | 60.2 | 53.0 | 49.6 |
| 120 | 36.8 | 41.4 | 53.2 | 65.5 | 75.4 | 83.5 | 92.3 | 97.0 | 100.1 | 98.9 | 98.8 | 95.9 | 93.3 | 84.0 | 75.1 | 67.6 | 54.3 | 44.6 | 39.1 |
| 125 | 29.2 | 34.8 | 48.9 | 59.9 | 71.3 | 81.6 | 88.3 | 93.8 | 95.3 | 95.3 | 96.0 | 89.9 | 86.0 | 79.3 | 69.2 | 63.8 | 49.2 | 35.5 | 30.3 |
| 130 | 19.8 | 27.3 | 41.9 | 54.5 | 61.9 | 75.7 | 82.5 | 87.9 | 88.8 | 85.1 | 88.5 | 86.6 | 82.4 | 72.2 | 65.2 | 53.5 | 42.9 | 29.0 | 22.0 |
| 135 | 15.0 | 21.7 | 35.9 | 48.6 | 58.3 | 68.2 | 76.7 | 80.5 | 83.1 | 83.5 | 82.4 | 81.8 | 73.8 | 65.1 | 59.7 | 49.0 | 37.6 | 23.5 | 15.3 |
| 140 | 8.6 | 16.4 | 27.8 | 42.4 | 52.7 | 63.7 | 70.4 | 74.1 | 77.4 | 79.2 | 80.3 | 74.4 | 69.6 | 62.7 | 51.5 | 45.0 | 30.6 | 18.8 | 9.9 |
| 145 | 4.2 | 11.2 | 25.6 | 39.5 | 49.2 | 58.3 | 65.7 | 70.2 | 75.0 | 72.9 | 72.1 | 70.0 | 65.1 | 57.1 | 46.9 | 36.4 | 24.9 | 13.1 | 5.2 |
| 150 | 2.7 | 8.7 | 21.9 | 32.7 | 44.1 | 56.3 | 62.0 | 64.6 | 67.5 | 66.7 | 68.2 | 70.0 | 57.1 | 51.9 | 42.9 | 33.5 | 20.2 | 10.0 | 3.6 |
| 155 | 2.5 | 12.0 | 22.8 | 36.9 | 48.9 | 58.5 | 64.9 | 72.3 | 74.5 | 74.1 | 72.2 | 66.9 | 60.9 | 52.7 | 41.3 | 28.6 | 15.8 | 4.4 | 1.1 |
| 160 | 2.1 | 12.4 | 25.8 | 38.7 | 49.4 | 58.7 | 62.7 | 70.5 | 75.8 | 73.8 | 73.7 | 68.8 | 61.6 | 50.9 | 42.2 | 29.6 | 16.8 | 5.4 | 1.3 |
| 165 | 2.3 | 11.8 | 25.3 | 38.3 | 49.3 | 58.2 | 65.7 | 72.3 | 73.4 | 73.8 | 71.1 | 69.4 | 61.3 | 53.2 | 41.5 | 30.6 | 16.4 | 4.9 | 1.1 |
| 170 | 2.2 | 12.1 | 25.2 | 37.4 | 48.9 | 56.4 | 63.6 | 71.2 | 72.2 | 74.6 | 71.9 | 65.5 | 59.9 | 52.5 | 40.9 | 30.3 | 16.9 | 5.3 | 1.2 |
| 175 | 2.2 | 11.6 | 24.8 | 37.4 | 48.1 | 55.9 | 64.2 | 69.9 | 72.2 | 70.7 | 69.7 | 66.8 | 60.6 | 51.4 | 40.4 | 29.0 | 16.7 | 5.4 | 1.2 |
| 180 | 3.8 | 12.5 | 25.2 | 35.6 | 46.2 | 53.5 | 63.1 | 65.9 | 69.6 | 69.6 | 68.4 | 63.9 | 58.0 | 48.5 | 40.2 | 27.4 | 15.3 | 4.4 | 1.5 |

Table 2a. Luminous intensity values, azimuth 0-180°

|  | 190 | 200 | 210 | 220 | 230 | 240 | 250 | 260 | 270 | 280 | 290 | 300 | 310 | 320 | 330 | 340 | 350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 | 142.7 |
| 5 | 143.8 | 144.2 | 144.0 | 144.6 | 145.3 | 146.2 | 147.1 | 147.7 | 147.2 | 146.3 | 145.5 | 144.8 | 144.2 | 143.8 | 143.9 | 144.2 | 144.0 |
| 10 | 143.0 | 143.8 | 143.5 | 143.4 | 144.3 | 145.4 | 146.5 | 145.9 | 146.2 | 147.7 | 147.6 | 145.6 | 145.6 | 145.0 | 143.9 | 142.1 | 141.8 |
| 15 | 145.7 | 142.6 | 142.9 | 142.8 | 143.1 | 145.0 | 147.2 | 150.3 | 152.2 | 154.3 | 149.4 | 149.0 | 147.1 | 143.4 | 144.8 | 145.4 | 143.2 |
| 20 | 141.9 | 141.8 | 145.0 | 145.5 | 145.4 | 150.9 | 148.1 | 151.9 | 150.7 | 148.8 | 149.7 | 147.8 | 145.9 | 144.5 | 145.1 | 144.7 | 140.1 |
| 25 | 139.8 | 142.4 | 141.9 | 145.4 | 146.6 | 150.1 | 150.8 | 150.0 | 149.2 | 150.7 | 153.2 | 154.3 | 146.1 | 148.2 | 146.4 | 141.7 | 141.2 |
| 30 | 131.4 | 139.4 | 140.6 | 145.4 | 146.8 | 152.0 | 151.4 | 155.2 | 155.5 | 154.4 | 149.4 | 152.9 | 148.4 | 143.4 | 148.2 | 139.0 | 137.6 |
| 35 | 132.0 | 141.1 | 138.3 | 144.6 | 148.3 | 151.2 | 152.6 | 154.4 | 155.7 | 153.1 | 155.3 | 152.3 | 150.7 | 149.5 | 141.8 | 139.0 | 139.2 |
| 40 | 126.1 | 141.3 | 140.0 | 147.8 | 147.3 | 149.7 | 153.7 | 155.9 | 154.6 | 154.8 | 152.7 | 152.4 | 152.7 | 143.8 | 141.6 | 133.7 | 134.6 |
| 45 | 124.9 | 137.3 | 136.9 | 143.8 | 146.7 | 148.3 | 153.6 | 153.7 | 157.1 | 153.0 | 153.5 | 151.9 | 149.0 | 145.0 | 141.0 | 131.5 | 131.0 |
| 50 | 121.9 | 130.9 | 137.6 | 137.8 | 144.6 | 149.6 | 152.8 | 152.9 | 155.7 | 156.2 | 153.3 | 153.1 | 150.0 | 142.2 | 133.7 | 134.5 | 124.5 |
| 55 | 117.1 | 123.3 | 135.1 | 136.2 | 146.0 | 148.9 | 153.0 | 156.2 | 155.9 | 157.2 | 151.9 | 148.1 | 148.7 | 142.0 | 136.6 | 130.6 | 125.9 |
| 60 | 111.4 | 119.8 | 133.1 | 132.4 | 139.4 | 145.4 | 148.5 | 150.5 | 152.2 | 151.1 | 150.4 | 147.8 | 147.8 | 138.4 | 131.7 | 125.4 | 117.2 |
| 65 | 110.4 | 115.1 | 128.9 | 136.2 | 138.7 | 137.4 | 148.3 | 150.9 | 148.6 | 153.8 | 150.2 | 147.4 | 143.0 | 131.8 | 129.4 | 121.4 | 115.0 |
| 70 | 103.9 | 109.9 | 123.1 | 135.3 | 136.2 | 142.6 | 146.7 | 146.4 | 147.0 | 147.4 | 145.4 | 142.9 | 139.0 | 133.7 | 126.6 | 116.3 | 112.5 |
| 75 | 102.1 | 106.7 | 117.4 | 131.0 | 131.0 | 137.0 | 140.4 | 145.4 | 149.1 | 146.3 | 146.5 | 140.7 | 138.3 | 127.3 | 121.4 | 113.9 | 104.9 |
| 80 | 96.0 | 102.4 | 110.4 | 123.1 | 133.5 | 137.3 | 138.6 | 142.0 | 141.8 | 142.4 | 137.8 | 138.7 | 130.8 | 125.4 | 114.7 | 107.4 | 95.8 |
| 85 | 89.6 | 96.7 | 106.6 | 119.7 | 126.1 | 132.7 | 138.1 | 136.7 | 139.7 | 138.9 | 139.6 | 131.6 | 129.7 | 121.3 | 111.1 | 103.9 | 93.1 |
| 90 | 85.4 | 90.7 | 98.4 | 110.3 | 123.9 | 126.2 | 134.3 | 134.0 | 137.9 | 133.0 | 133.6 | 130.7 | 122.0 | 113.7 | 106.0 | 95.5 | 84.8 |
| 95 | 79.5 | 87.0 | 94.8 | 104.4 | 117.9 | 121.8 | 129.6 | 132.5 | 132.1 | 126.9 | 130.1 | 125.2 | 119.5 | 111.9 | 101.7 | 91.2 | 76.6 |
| 100 | 71.9 | 79.7 | 90.0 | 101.2 | 111.1 | 120.0 | 120.8 | 123.1 | 127.5 | 127.1 | 123.5 | 119.3 | 115.1 | 105.9 | 97.7 | 83.8 | 76.2 |
| 105 | 63.2 | 75.8 | 83.7 | 95.1 | 100.8 | 111.6 | 118.7 | 122.2 | 123.1 | 118.3 | 115.8 | 113.0 | 104.6 | 96.7 | 86.0 | 78.8 | 65.9 |
| 110 | 58.9 | 68.1 | 76.0 | 85.7 | 97.7 | 105.6 | 111.2 | 112.5 | 117.2 | 115.8 | 114.3 | 108.9 | 99.5 | 88.3 | 80.8 | 70.5 | 58.7 |
| 115 | 51.9 | 58.9 | 70.4 | 80.5 | 95.1 | 101.1 | 105.2 | 110.6 | 111.3 | 108.7 | 109.3 | 102.3 | 94.4 | 85.9 | 76.1 | 63.5 | 51.9 |
| 120 | 45.3 | 54.2 | 65.9 | 76.3 | 87.6 | 92.1 | 100.4 | 106.9 | 105.5 | 107.3 | 104.2 | 100.2 | 88.2 | 82.0 | 68.6 | 57.6 | 44.3 |
| 125 | 34.9 | 46.9 | 58.9 | 69.0 | 81.8 | 88.4 | 89.3 | 93.4 | 98.7 | 98.8 | 95.6 | 89.0 | 83.1 | 74.7 | 61.4 | 50.2 | 37.6 |
| 130 | 28.3 | 39.8 | 54.5 | 64.9 | 73.2 | 79.6 | 87.4 | 89.9 | 93.1 | 91.5 | 88.5 | 84.2 | 75.9 | 67.2 | 57.4 | 41.7 | 28.4 |
| 135 | 21.5 | 33.6 | 45.8 | 60.4 | 69.0 | 75.3 | 79.1 | 82.0 | 86.9 | 90.9 | 83.3 | 77.6 | 73.2 | 60.5 | 50.8 | 37.2 | 23.5 |
| 140 | 16.8 | 29.4 | 40.9 | 52.6 | 60.5 | 71.0 | 77.2 | 75.9 | 81.3 | 82.3 | 75.9 | 71.9 | 65.7 | 54.2 | 45.3 | 30.8 | 18.1 |
| 145 | 13.1 | 23.6 | 35.5 | 47.9 | 56.5 | 64.0 | 72.0 | 75.5 | 75.4 | 73.4 | 71.8 | 66.5 | 60.4 | 51.0 | 36.5 | 25.6 | 11.5 |
| 150 | 9.1 | 21.9 | 31.5 | 46.4 | 50.6 | 62.0 | 69.1 | 69.2 | 73.3 | 73.1 | 64.7 | 60.8 | 58.8 | 47.3 | 36.3 | 22.9 | 10.2 |
| 155 | 11.8 | 23.9 | 35.0 | 47.7 | 58.1 | 63.3 | 67.0 | 72.7 | 73.6 | 69.9 | 65.7 | 60.9 | 49.7 | 37.4 | 26.1 | 15.8 | 3.9 |
| 160 | 12.0 | 24.5 | 37.9 | 49.4 | 57.7 | 63.6 | 68.2 | 73.6 | 72.9 | 71.2 | 67.5 | 61.6 | 52.2 | 39.9 | 28.3 | 15.8 | 4.6 |
| 165 | 12.0 | 24.4 | 38.6 | 49.8 | 56.8 | 63.3 | 68.3 | 73.7 | 74.1 | 72.2 | 68.2 | 61.9 | 50.7 | 39.9 | 27.8 | 15.1 | 3.7 |
| 170 | 11.5 | 24.7 | 37.5 | 48.4 | 57.0 | 64.3 | 68.1 | 71.4 | 73.2 | 71.4 | 66.5 | 60.1 | 52.1 | 40.9 | 28.7 | 14.6 | 3.8 |
| 175 | 11.2 | 23.8 | 36.8 | 48.7 | 56.9 | 62.2 | 68.4 | 72.6 | 71.8 | 71.0 | 66.2 | 59.4 | 50.8 | 40.4 | 27.7 | 15.0 | 4.1 |
| 180 | 10.7 | 21.6 | 35.1 | 46.4 | 54.1 | 59.9 | 67.6 | 70.1 | 70.8 | 68.5 | 64.0 | 57.2 | 49.5 | 39.2 | 28.2 | 15.4 | 5.1 |

Table 2b. Luminous intensity values, azimuth 190-350°

Signature:

_________________________________

Print Name:

GH JOHN

_________________________________

Date:

26-02-2013

_________________________________

Partner / Director
Duly authorised to sign on behalf of:

Photometric and Optical Testing Services LLP