| Photometric Test Report |  |  |
| --- | --- | --- |
| Report Number: POTS/GJ13087 | Report Date: 06-03-2013 | Prepared By: G John |
| Test Laboratory: Photometric and Optical Testing Services, Cotswold Business Centre, 42 A P Ellis Road, Rissington Business Park, Upper Rissington, Gloucestershire, GL54 2QB |  |  |
| Company Registration Number: Registered in England & Wales No. OC352911 |  |  |
| Registered Address: Thistle Down Barn, Holcot Lane, Sywell, Northampton, NN6 0BG |  |  |

| Client Details |  |
| --- | --- |
| Company: Astro Lighting | Email: technical@astrolighting.co.uk |
| Address: Astro Lighting Limited, G2 River Way, Harlow CM20 2DP, Great Britain |  |

| Details of Product Tested |  |
| --- | --- |
| Manufacturer: Astro Lighting | Source Type: Domestic light |
| Model: RIO 325 | Serial Number: 7172 |
| Lamp Type: LED |  |
| Power Supply Used: Uninterruptible AC power supply |  |
| Voltage(AC V) = 240.5 | Current (mA)= 81 |
| Power (Watts)= 14.70 | Power factor= 0.755 |

| Integrating Sphere Test |  |  |
| --- | --- | --- |
| Date of Test: 13-02-2013 |  | Ambient Temperature:25°C |
| Measurement Filename: 7172 RIO 325 |  |  |
| Instrument Used: Labsphere model CSLMS HALOGEN 4060 integrating sphere spectroradiometer |  |  |
| Integrating Sphere Size: 1m |  | Measurement Geometry (2 / 4): 4 |
| Sample Orientation: Horizontal |  | Auxiliary Correction Applied: YES |
| Comments: |  |  |
| Date of Last Calibration (Operating Hours): 08-11-2012 (02:36) |  | Spectral Flux Standard Lamp Used: SCL-1400 |
| Standard Lamp Serial Number: K75 |  | Traceable: to NIST standards |
| Calibration Certificate Number: DM-02008-001 |  | Calibration Certificate Date: 19th February 2010 |
| Calibration Lamp Uncertainty: ± 0.67% (k=2) |  |  |
| Results |  |  |
| Flux (lumens): 1198 |  |  |
| CIE 1931 Chromaticity Cx: 0.4311 |  | CIE 1931 Chromaticity Cy: 0.3983 |
| CRI (%): 80.04 |  | CCT (K): 3050 |

Figure 1: Spectral Irradiance

Figure 2: CIE 1931 diagram.

| Goniophotometer Test |  |  |  |
| --- | --- | --- | --- |
| Date of Test: 13-02-2013 |  | Ambient Temperature: 25°C |  |
| Measurement Filename: 7172 RIO 325 |  |  |  |
| Instrument Used: Radiant Imaging NFMS0800 Goniometer with ProMetric PM-1200N-1 Imaging Photometer |  |  |  |
| Photometer Working Distance:2.0 m |  | Measurement Geometry: Near-Field |  |
| Comments: |  |  |  |
| Reference Photometer Used: Specbos1201 |  | Reference Photometer Serial Number: 2911670 |  |
| Traceable: to NPL standards, UKAS Accredited |  | Calibration Certificate Number: 121104 |  |
| Calibration Certificate Date: 25th October 2011 |  | Sample Stabilisation Time (minutes):30 |  |
| Reference Photometer Calibration Uncertainty: ± 2.4% (k=2, 20-200 lux, CIE illuminant A source) |  |  |  |
| Scan Set Up |  |  |  |
| Direction | Range |  | Increment |
| Inclination Zone 1 | 0-180° |  | 3° |
| Azimuth | 0-360° |  | 10° |
| Results |  |  |  |
| Integrated Luminous Flux (lumens):1198.0 | Peak Intensity (3° Spot, candelas): 282.0 |  | Efficacy (lumens/Watt): 80.02 |
| Beam Angle (50% of max intensity C0-180, degrees): 86.7 |  |  |  |
| Photometric Filename (IES LM-63-2002): 7172 RIO 325 |  |  |  |
| IES File – Absolute or Relative Format? Absolute |  |  |  |
| Photometric Filename (EULUMDAT): 7172 RIO 325 |  |  |  |
| EULUMDAT File – Absolute or Relative Format? Absolute |  |  |  |

Figure 3: Far-Field Luminous Intensity (C0-180, Cartesian Coordinates)

Figure 4: Far-Field Luminous Intensity (C0-180, C90-270, Polar Coordinates)

Figure 4. Illuminance cone diagram.

Table 1. UGR values

|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 |
| 3 | 149.8 | 149.6 | 149.6 | 147.5 | 148.4 | 148.2 | 147.0 | 145.9 | 145.5 | 146.3 | 147.2 | 147.5 | 146.7 | 145.2 | 143.9 | 143.3 | 143.5 | 143.9 | 144.8 |
| 6 | 150.7 | 148.9 | 148.9 | 151.3 | 152.8 | 152.7 | 148.7 | 144.6 | 144.0 | 143.4 | 142.1 | 143.3 | 143.9 | 144.2 | 146.5 | 148.2 | 150.6 | 152.0 | 146.4 |
| 9 | 147.1 | 149.3 | 148.7 | 153.7 | 154.1 | 159.1 | 155.1 | 150.2 | 145.6 | 141.0 | 138.8 | 138.1 | 144.8 | 145.0 | 145.3 | 150.8 | 155.0 | 160.3 | 156.8 |
| 12 | 159.2 | 160.3 | 156.1 | 152.5 | 150.5 | 151.2 | 153.9 | 152.5 | 146.6 | 139.2 | 139.5 | 141.7 | 141.2 | 152.3 | 157.3 | 159.0 | 169.6 | 169.3 | 168.7 |
| 15 | 164.4 | 168.1 | 161.5 | 157.6 | 161.6 | 154.5 | 150.1 | 149.0 | 144.3 | 138.7 | 137.4 | 141.5 | 147.6 | 158.2 | 168.6 | 174.4 | 175.0 | 178.6 | 177.8 |
| 18 | 176.6 | 172.8 | 171.5 | 168.3 | 157.0 | 149.6 | 154.0 | 144.3 | 143.4 | 131.9 | 135.6 | 140.1 | 154.7 | 162.7 | 172.3 | 175.8 | 178.8 | 180.1 | 179.3 |
| 21 | 179.4 | 178.6 | 180.3 | 169.1 | 159.1 | 156.3 | 148.5 | 137.8 | 140.0 | 130.8 | 129.9 | 142.4 | 157.9 | 163.9 | 170.9 | 174.2 | 178.8 | 177.5 | 178.6 |
| 24 | 192.5 | 189.7 | 178.9 | 172.0 | 168.0 | 159.5 | 148.4 | 140.6 | 138.5 | 126.9 | 137.7 | 146.3 | 153.5 | 169.8 | 166.5 | 169.3 | 179.9 | 178.4 | 185.6 |
| 27 | 193.6 | 192.0 | 192.0 | 181.7 | 171.3 | 162.2 | 147.2 | 136.8 | 132.0 | 123.0 | 124.2 | 152.1 | 157.1 | 170.1 | 159.9 | 174.5 | 179.7 | 184.4 | 191.4 |
| 30 | 196.8 | 199.7 | 193.9 | 174.9 | 177.0 | 163.5 | 141.1 | 130.9 | 129.8 | 119.1 | 127.7 | 146.5 | 156.5 | 152.8 | 163.3 | 173.4 | 181.4 | 187.2 | 198.4 |
| 33 | 211.7 | 204.3 | 191.9 | 193.4 | 176.1 | 150.5 | 144.0 | 136.3 | 125.5 | 117.0 | 122.3 | 144.0 | 155.7 | 154.3 | 161.4 | 183.1 | 187.3 | 193.8 | 206.4 |
| 36 | 212.1 | 213.8 | 214.6 | 189.6 | 170.3 | 162.8 | 151.3 | 130.2 | 125.8 | 111.7 | 122.3 | 141.9 | 144.7 | 148.6 | 168.4 | 181.8 | 185.6 | 206.5 | 208.2 |
| 39 | 221.1 | 217.0 | 198.1 | 193.1 | 184.0 | 161.2 | 138.4 | 127.2 | 118.7 | 109.3 | 115.7 | 140.1 | 136.3 | 147.2 | 170.4 | 186.1 | 188.8 | 206.7 | 221.1 |
| 42 | 221.4 | 226.2 | 216.0 | 205.4 | 179.4 | 154.2 | 139.3 | 123.6 | 110.3 | 101.7 | 119.1 | 129.2 | 130.7 | 148.7 | 174.8 | 181.1 | 195.6 | 208.2 | 224.5 |
| 45 | 235.1 | 232.7 | 209.2 | 189.7 | 174.4 | 161.0 | 143.2 | 126.5 | 103.2 | 93.4 | 118.1 | 116.5 | 126.9 | 151.7 | 178.6 | 185.2 | 193.4 | 217.9 | 226.8 |
| 48 | 237.2 | 230.5 | 208.2 | 198.5 | 176.9 | 162.7 | 136.3 | 111.2 | 96.6 | 90.2 | 102.5 | 115.1 | 133.0 | 154.6 | 175.0 | 185.9 | 199.3 | 218.2 | 230.2 |
| 51 | 242.9 | 235.9 | 212.5 | 202.4 | 180.2 | 161.7 | 137.2 | 110.5 | 95.4 | 82.2 | 88.1 | 99.4 | 127.7 | 153.6 | 172.3 | 179.8 | 194.3 | 222.7 | 238.3 |
| 54 | 237.8 | 235.4 | 228.9 | 206.2 | 181.2 | 151.1 | 124.9 | 104.2 | 86.1 | 79.8 | 86.1 | 101.5 | 126.3 | 148.0 | 170.7 | 185.9 | 209.7 | 233.9 | 245.4 |
| 57 | 245.5 | 250.8 | 234.8 | 213.7 | 179.0 | 148.5 | 118.9 | 97.2 | 77.9 | 74.7 | 81.0 | 93.0 | 120.6 | 142.9 | 164.6 | 182.8 | 205.4 | 237.5 | 252.2 |
| 60 | 259.6 | 249.9 | 221.5 | 191.8 | 167.8 | 142.1 | 119.1 | 97.6 | 73.7 | 66.0 | 76.5 | 87.9 | 112.1 | 134.4 | 158.4 | 182.9 | 219.0 | 247.9 | 265.3 |
| 63 | 268.1 | 254.3 | 224.0 | 191.5 | 171.7 | 140.1 | 112.6 | 88.0 | 66.0 | 57.5 | 67.7 | 81.1 | 106.9 | 131.5 | 155.8 | 186.5 | 214.8 | 248.2 | 262.5 |
| 66 | 248.5 | 250.5 | 233.6 | 206.4 | 181.5 | 149.0 | 107.9 | 76.0 | 59.1 | 52.9 | 58.8 | 72.5 | 99.5 | 125.9 | 152.7 | 188.0 | 223.3 | 264.9 | 268.9 |
| 69 | 245.5 | 245.3 | 222.6 | 209.1 | 171.6 | 142.8 | 101.9 | 72.9 | 52.3 | 45.4 | 51.9 | 64.6 | 96.4 | 121.1 | 150.2 | 189.9 | 228.5 | 263.4 | 282.1 |
| 72 | 271.7 | 260.3 | 240.9 | 213.2 | 172.0 | 136.4 | 97.9 | 63.1 | 46.8 | 37.5 | 41.7 | 56.0 | 83.0 | 113.3 | 155.7 | 199.8 | 234.3 | 268.8 | 281.6 |
| 75 | 264.3 | 259.1 | 229.6 | 203.4 | 168.6 | 130.4 | 81.8 | 49.7 | 33.7 | 28.9 | 30.8 | 42.2 | 72.7 | 116.0 | 154.8 | 189.2 | 237.1 | 270.3 | 278.3 |
| 78 | 260.7 | 251.2 | 223.1 | 199.1 | 167.2 | 118.3 | 75.4 | 41.1 | 27.9 | 21.7 | 23.4 | 35.8 | 69.0 | 113.7 | 156.8 | 195.7 | 235.3 | 256.7 | 273.4 |
| 81 | 261.2 | 249.8 | 222.8 | 194.1 | 160.2 | 119.5 | 77.1 | 35.3 | 16.2 | 15.3 | 14.9 | 28.3 | 67.7 | 109.9 | 149.7 | 196.3 | 227.4 | 256.0 | 270.8 |
| 84 | 256.9 | 251.0 | 221.2 | 193.7 | 159.7 | 116.4 | 73.1 | 31.9 | 11.7 | 8.1 | 10.0 | 24.4 | 66.1 | 104.0 | 139.3 | 186.6 | 225.0 | 243.1 | 261.8 |
| 87 | 241.8 | 246.0 | 227.7 | 188.1 | 152.3 | 112.6 | 70.6 | 29.5 | 8.7 | 4.7 | 6.8 | 21.1 | 46.7 | 81.0 | 111.7 | 154.4 | 204.1 | 228.2 | 273.8 |
| 90 | 240.6 | 245.3 | 224.2 | 185.2 | 150.2 | 108.1 | 63.2 | 26.2 | 7.7 | 2.1 | 5.1 | 10.8 | 24.9 | 60.7 | 105.2 | 143.9 | 185.9 | 230.6 | 257.9 |
| 93 | 236.1 | 225.1 | 202.1 | 164.2 | 128.1 | 90.1 | 52.2 | 21.8 | 4.8 | 0.7 | 2.0 | 6.0 | 19.3 | 53.4 | 100.1 | 142.2 | 190.7 | 218.7 | 235.7 |
| 96 | 229.2 | 220.1 | 195.1 | 158.1 | 123.1 | 82.8 | 39.6 | 10.3 | 4.5 | 0.8 | 0.8 | 6.2 | 20.3 | 46.4 | 92.1 | 130.5 | 179.9 | 217.2 | 230.8 |
| 99 | 220.5 | 207.2 | 184.6 | 142.1 | 101.8 | 42.1 | 14.4 | 7.4 | 3.7 | 0.7 | 0.6 | 4.7 | 9.8 | 30.9 | 84.2 | 122.1 | 174.4 | 208.4 | 219.8 |
| 102 | 205.9 | 184.3 | 126.7 | 55.5 | 34.9 | 16.5 | 7.2 | 6.0 | 2.8 | 0.6 | 0.5 | 3.8 | 7.0 | 12.1 | 23.6 | 39.2 | 97.1 | 145.8 | 145.0 |
| 105 | 108.7 | 88.8 | 38.1 | 12.6 | 6.4 | 6.4 | 5.5 | 4.4 | 2.2 | 0.5 | 0.6 | 3.3 | 6.0 | 6.3 | 6.3 | 9.9 | 27.6 | 43.9 | 30.4 |
| 108 | 9.7 | 8.1 | 5.1 | 5.7 | 4.7 | 5.2 | 4.6 | 3.5 | 1.8 | 0.6 | 1.3 | 2.9 | 4.7 | 5.4 | 5.3 | 4.5 | 4.2 | 4.2 | 2.3 |
| 111 | 4.9 | 3.6 | 2.5 | 4.9 | 4.0 | 4.3 | 3.4 | 2.8 | 1.5 | 0.6 | 1.2 | 2.5 | 3.9 | 4.4 | 4.4 | 3.6 | 3.1 | 2.5 | 1.9 |
| 114 | 1.9 | 1.3 | 1.5 | 2.4 | 2.9 | 3.3 | 3.1 | 2.3 | 1.1 | 0.5 | 0.8 | 2.2 | 3.7 | 4.3 | 3.8 | 3.2 | 2.5 | 1.6 | 0.7 |
| 117 | 0.9 | 0.8 | 1.1 | 2.0 | 2.5 | 2.6 | 2.5 | 1.8 | 1.0 | 0.4 | 0.7 | 1.8 | 3.2 | 3.6 | 3.6 | 2.8 | 1.9 | 1.1 | 0.5 |
| 120 | 0.8 | 0.4 | 1.0 | 1.5 | 2.0 | 2.1 | 2.0 | 1.4 | 0.8 | 0.5 | 0.7 | 1.7 | 2.7 | 2.9 | 2.9 | 2.4 | 1.6 | 0.9 | 0.4 |
| 123 | 0.6 | 0.4 | 0.7 | 1.2 | 1.7 | 1.9 | 1.6 | 1.2 | 0.6 | 0.4 | 0.6 | 1.3 | 2.2 | 2.5 | 2.5 | 2.1 | 1.6 | 0.9 | 0.4 |
| 126 | 0.6 | 0.4 | 0.5 | 1.0 | 1.2 | 1.4 | 1.3 | 1.0 | 0.5 | 0.5 | 0.6 | 1.3 | 1.7 | 2.2 | 2.1 | 1.8 | 1.3 | 0.8 | 0.6 |
| 129 | 0.4 | 0.1 | 0.5 | 0.8 | 1.0 | 1.1 | 1.0 | 0.7 | 0.4 | 0.1 | 0.5 | 0.9 | 1.4 | 1.9 | 1.9 | 1.8 | 1.1 | 0.7 | 0.3 |
| 132 | 0.6 | 0.4 | 0.4 | 0.5 | 0.8 | 0.8 | 0.8 | 0.6 | 0.4 | 0.0 | 0.5 | 0.8 | 1.2 | 1.5 | 1.6 | 1.5 | 1.0 | 0.5 | 0.1 |
| 135 | 0.2 | 0.1 | 0.4 | 0.5 | 0.6 | 0.7 | 0.6 | 0.5 | 0.5 | 0.0 | 0.4 | 0.7 | 1.0 | 1.4 | 1.3 | 1.0 | 0.8 | 0.5 | 0.1 |
| 138 | 0.4 | 0.1 | 0.1 | 0.4 | 0.4 | 0.6 | 0.5 | 0.5 | 0.1 | 0.1 | 0.4 | 0.7 | 0.8 | 1.0 | 0.9 | 0.9 | 0.7 | 0.4 | 0.0 |
| 141 | 0.3 | 0.0 | 0.1 | 0.1 | 0.4 | 0.5 | 0.4 | 0.4 | 0.2 | 0.1 | 0.4 | 0.5 | 0.7 | 0.9 | 1.0 | 0.8 | 0.8 | 0.4 | 0.0 |
| 144 | 0.4 | 0.0 | 0.0 | 0.1 | 0.1 | 0.4 | 0.4 | 0.4 | 0.5 | 0.2 | 0.1 | 0.4 | 0.6 | 0.7 | 0.7 | 1.3 | 3.7 | 1.1 | 0.5 |
| 147 | 0.4 | 0.0 | 0.0 | 0.1 | 0.3 | 0.0 | 0.5 | 0.1 | 0.1 | 0.0 | 0.2 | 0.4 | 0.5 | 0.6 | 0.6 | 3.0 | 4.9 | 4.8 | 2.7 |
| 150 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.2 | 0.0 | 0.1 | 0.1 | 0.2 | 0.2 | 0.4 | 0.4 | 0.7 | 4.4 | 4.7 | 4.6 | 3.5 |
| 153 | 0.1 | 0.0 | 0.1 | 0.0 | 0.4 | 0.1 | 0.2 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 | 0.4 | 1.6 | 4.9 | 4.8 | 4.5 | 3.5 |
| 156 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.2 | 2.6 | 5.1 | 5.1 | 5.0 | 4.3 | 3.8 |
| 159 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 | 0.5 | 1.4 | 52.3 | 76.3 | 26.5 | 13.7 | 6.0 | 4.0 |
| 162 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 1.4 | 4.6 | 30.3 | 65.5 | 42.3 | 59.3 | 5.9 | 3.4 |
| 165 | 0.0 | 0.0 | 0.2 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 1.5 | 6.2 | 6.5 | 8.2 | 18.9 | 15.4 | 5.1 | 3.0 |
| 168 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.9 | 3.7 | 6.0 | 6.6 | 7.2 | 5.9 | 4.7 | 3.2 |
| 171 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.4 | 2.9 | 4.6 | 5.5 | 5.2 | 4.6 | 3.4 |
| 174 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.5 | 0.6 | 0.8 | 1.2 | 1.9 | 2.5 | 1.9 |
| 177 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 180 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 2a. Luminous intensity values, azimuth 0-180°

|  | 190 | 200 | 210 | 220 | 230 | 240 | 250 | 260 | 270 | 280 | 290 | 300 | 310 | 320 | 330 | 340 | 350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 | 146.1 |
| 3 | 145.6 | 146.1 | 145.8 | 145.5 | 143.4 | 141.9 | 141.9 | 143.5 | 144.9 | 146.1 | 146.7 | 147.9 | 149.6 | 150.3 | 149.7 | 150.6 | 150.1 |
| 6 | 145.2 | 147.7 | 147.8 | 145.5 | 142.8 | 143.7 | 148.5 | 150.8 | 149.6 | 149.1 | 150.6 | 151.6 | 153.6 | 155.8 | 155.6 | 154.9 | 153.4 |
| 9 | 159.8 | 159.5 | 156.3 | 151.6 | 151.2 | 154.0 | 154.2 | 151.3 | 146.3 | 149.1 | 154.2 | 154.8 | 158.1 | 159.2 | 161.3 | 158.3 | 151.5 |
| 12 | 172.3 | 172.9 | 170.3 | 160.1 | 155.8 | 151.8 | 148.5 | 150.4 | 154.8 | 151.9 | 155.4 | 161.6 | 158.9 | 157.8 | 157.4 | 160.6 | 159.2 |
| 15 | 177.9 | 179.0 | 174.4 | 171.8 | 166.6 | 155.5 | 152.3 | 145.5 | 147.7 | 147.8 | 162.6 | 163.4 | 158.3 | 158.7 | 164.0 | 162.4 | 162.4 |
| 18 | 174.1 | 185.4 | 181.8 | 181.5 | 171.1 | 166.9 | 153.5 | 147.7 | 147.1 | 152.7 | 152.4 | 160.8 | 153.5 | 167.0 | 166.5 | 170.3 | 175.6 |
| 21 | 171.5 | 181.8 | 181.0 | 184.7 | 181.3 | 167.8 | 152.8 | 151.3 | 145.4 | 155.0 | 151.4 | 155.6 | 161.2 | 167.1 | 177.4 | 179.0 | 182.9 |
| 24 | 177.9 | 184.1 | 178.7 | 174.7 | 175.6 | 173.2 | 157.2 | 147.7 | 145.7 | 149.6 | 153.6 | 153.2 | 165.5 | 176.0 | 178.8 | 186.0 | 190.3 |
| 27 | 185.7 | 186.0 | 178.1 | 174.8 | 182.6 | 171.3 | 165.4 | 143.7 | 138.8 | 147.8 | 145.5 | 162.2 | 169.3 | 170.6 | 189.2 | 193.1 | 198.5 |
| 30 | 189.2 | 187.5 | 186.3 | 175.5 | 169.6 | 171.8 | 156.0 | 142.4 | 135.8 | 148.0 | 155.5 | 153.0 | 173.8 | 174.7 | 183.6 | 198.3 | 202.4 |
| 33 | 198.4 | 192.2 | 194.2 | 181.0 | 166.5 | 165.9 | 159.8 | 145.0 | 136.8 | 145.8 | 155.6 | 155.9 | 161.3 | 185.5 | 192.0 | 196.4 | 208.5 |
| 36 | 205.2 | 193.9 | 187.0 | 181.4 | 167.6 | 162.7 | 159.1 | 139.9 | 132.2 | 146.7 | 141.5 | 160.6 | 167.0 | 173.0 | 197.5 | 209.4 | 212.2 |
| 39 | 211.3 | 192.1 | 194.1 | 184.1 | 168.4 | 155.9 | 157.6 | 139.9 | 128.1 | 137.0 | 136.4 | 149.7 | 173.2 | 190.9 | 188.4 | 203.4 | 222.3 |
| 42 | 221.7 | 199.4 | 191.4 | 185.0 | 171.3 | 149.7 | 149.0 | 142.3 | 122.5 | 134.8 | 138.2 | 147.6 | 162.9 | 187.8 | 212.2 | 213.9 | 222.7 |
| 45 | 224.3 | 209.6 | 192.9 | 187.5 | 175.2 | 154.6 | 136.7 | 126.9 | 115.4 | 121.4 | 133.3 | 149.4 | 167.5 | 175.9 | 196.4 | 220.5 | 237.9 |
| 48 | 229.3 | 208.9 | 194.3 | 183.7 | 174.4 | 153.5 | 132.8 | 121.9 | 115.6 | 121.7 | 126.3 | 144.7 | 172.5 | 183.0 | 203.3 | 215.5 | 234.0 |
| 51 | 233.1 | 208.2 | 194.4 | 181.9 | 171.3 | 150.3 | 131.5 | 110.2 | 106.0 | 113.1 | 120.9 | 144.2 | 166.7 | 189.0 | 199.1 | 225.2 | 237.3 |
| 54 | 238.5 | 219.0 | 194.5 | 179.5 | 166.7 | 147.4 | 126.9 | 110.6 | 103.9 | 101.9 | 112.5 | 128.9 | 156.1 | 189.7 | 211.7 | 228.7 | 229.1 |
| 57 | 249.5 | 220.9 | 191.4 | 176.0 | 164.8 | 142.7 | 120.4 | 101.8 | 93.3 | 99.4 | 107.3 | 120.0 | 154.2 | 188.6 | 216.3 | 239.1 | 245.3 |
| 60 | 254.8 | 228.0 | 191.7 | 171.6 | 144.9 | 129.6 | 114.7 | 99.5 | 86.4 | 89.8 | 103.5 | 122.7 | 146.6 | 174.1 | 199.1 | 231.4 | 257.9 |
| 63 | 258.8 | 234.2 | 195.1 | 167.8 | 139.9 | 119.6 | 99.0 | 84.6 | 76.0 | 81.2 | 91.6 | 112.6 | 146.0 | 170.1 | 201.9 | 240.0 | 262.9 |
| 66 | 266.0 | 238.4 | 203.7 | 166.7 | 135.9 | 112.9 | 87.1 | 71.3 | 69.8 | 72.4 | 84.6 | 114.6 | 152.0 | 187.2 | 207.4 | 228.3 | 242.1 |
| 69 | 271.9 | 237.2 | 208.9 | 166.2 | 131.2 | 97.5 | 79.0 | 59.3 | 55.0 | 60.6 | 75.6 | 104.9 | 145.7 | 181.4 | 209.9 | 231.9 | 247.3 |
| 72 | 274.8 | 250.2 | 226.5 | 171.9 | 127.0 | 87.5 | 63.4 | 51.2 | 43.2 | 53.6 | 69.7 | 96.5 | 138.5 | 177.5 | 216.4 | 250.5 | 261.9 |
| 75 | 281.1 | 249.0 | 219.4 | 173.7 | 123.3 | 82.2 | 53.0 | 39.8 | 36.1 | 39.5 | 52.9 | 86.1 | 127.8 | 173.6 | 213.6 | 247.4 | 266.5 |
| 78 | 275.4 | 243.3 | 218.6 | 176.0 | 127.7 | 79.4 | 41.8 | 26.4 | 24.4 | 30.3 | 42.4 | 79.5 | 124.0 | 169.7 | 213.7 | 243.0 | 266.5 |
| 81 | 277.3 | 245.6 | 215.6 | 171.2 | 123.6 | 80.7 | 36.7 | 17.1 | 17.3 | 19.1 | 36.1 | 77.1 | 118.9 | 164.2 | 197.7 | 233.7 | 254.2 |
| 84 | 268.2 | 241.9 | 213.8 | 165.9 | 117.4 | 76.0 | 32.6 | 12.2 | 9.7 | 13.3 | 33.0 | 69.7 | 105.8 | 143.1 | 178.7 | 214.0 | 252.3 |
| 87 | 259.9 | 233.3 | 204.7 | 166.9 | 117.9 | 73.4 | 30.2 | 8.7 | 4.8 | 7.7 | 24.7 | 52.4 | 94.5 | 134.7 | 176.3 | 213.4 | 237.3 |
| 90 | 265.3 | 240.6 | 200.4 | 161.0 | 119.5 | 72.4 | 29.2 | 8.2 | 0.0 | 5.4 | 16.7 | 41.0 | 87.0 | 122.5 | 168.2 | 207.6 | 240.7 |
| 93 | 238.6 | 217.2 | 189.9 | 152.8 | 106.0 | 58.2 | 23.2 | 5.3 | 0.7 | 3.0 | 11.5 | 35.5 | 69.1 | 107.7 | 147.0 | 186.3 | 220.3 |
| 96 | 234.7 | 211.5 | 187.7 | 142.5 | 95.4 | 37.7 | 11.2 | 4.9 | 0.6 | 2.3 | 9.4 | 30.2 | 64.3 | 100.5 | 144.2 | 182.5 | 213.2 |
| 99 | 223.5 | 203.0 | 181.5 | 83.2 | 31.1 | 15.5 | 7.5 | 3.7 | 0.8 | 1.9 | 7.2 | 21.3 | 55.3 | 92.8 | 138.7 | 177.1 | 206.3 |
| 102 | 114.6 | 68.5 | 50.3 | 25.8 | 12.2 | 8.0 | 6.0 | 2.9 | 0.4 | 1.6 | 6.2 | 11.8 | 24.3 | 53.6 | 118.3 | 167.0 | 199.5 |
| 105 | 21.8 | 8.8 | 5.3 | 7.5 | 6.8 | 5.9 | 4.8 | 2.4 | 0.4 | 1.6 | 5.6 | 8.1 | 9.0 | 20.0 | 56.6 | 92.5 | 118.9 |
| 108 | 2.6 | 4.7 | 4.4 | 5.0 | 5.4 | 5.0 | 3.8 | 2.1 | 0.6 | 1.4 | 4.7 | 6.8 | 7.7 | 7.4 | 9.0 | 8.5 | 10.2 |
| 111 | 1.1 | 4.2 | 2.8 | 3.6 | 4.2 | 4.0 | 3.0 | 1.6 | 0.5 | 1.0 | 3.9 | 5.4 | 6.0 | 6.4 | 8.7 | 5.2 | 3.7 |
| 114 | 0.4 | 1.1 | 1.9 | 2.9 | 3.4 | 3.3 | 2.4 | 1.3 | 0.4 | 1.0 | 2.9 | 4.5 | 4.9 | 5.1 | 4.4 | 2.7 | 1.9 |
| 117 | 0.1 | 0.6 | 1.5 | 2.3 | 2.7 | 2.6 | 1.9 | 1.1 | 0.4 | 1.0 | 2.3 | 3.9 | 4.4 | 4.5 | 3.5 | 2.2 | 1.3 |
| 120 | 0.1 | 0.4 | 1.2 | 1.9 | 2.4 | 2.2 | 1.4 | 0.9 | 0.4 | 0.9 | 2.0 | 3.5 | 3.7 | 3.7 | 2.9 | 1.8 | 1.0 |
| 123 | 0.1 | 0.0 | 0.9 | 1.5 | 1.7 | 1.6 | 1.2 | 0.8 | 0.0 | 0.8 | 2.0 | 2.7 | 3.2 | 3.2 | 2.8 | 1.6 | 0.8 |
| 126 | 0.2 | 0.0 | 0.7 | 1.2 | 1.3 | 1.2 | 1.1 | 0.6 | 0.1 | 0.8 | 1.6 | 2.2 | 2.7 | 2.7 | 2.3 | 1.4 | 0.5 |
| 129 | 0.0 | 0.1 | 0.6 | 1.0 | 1.2 | 1.0 | 0.8 | 0.5 | 0.6 | 0.6 | 1.2 | 1.7 | 2.0 | 2.1 | 2.1 | 1.3 | 0.6 |
| 132 | 0.0 | 0.0 | 0.4 | 0.8 | 0.8 | 0.9 | 0.6 | 0.4 | 0.4 | 0.6 | 1.1 | 1.4 | 1.7 | 1.9 | 1.6 | 1.1 | 0.5 |
| 135 | 0.0 | 0.0 | 0.3 | 0.5 | 0.7 | 0.7 | 0.6 | 0.4 | 0.4 | 0.6 | 0.9 | 1.2 | 1.4 | 1.5 | 1.3 | 1.0 | 0.5 |
| 138 | 0.0 | 0.0 | 0.0 | 0.1 | 0.5 | 0.4 | 0.5 | 0.4 | 0.1 | 0.5 | 0.8 | 1.1 | 1.2 | 1.1 | 1.2 | 0.8 | 0.4 |
| 141 | 0.0 | 0.0 | 0.0 | 0.1 | 0.4 | 0.4 | 0.4 | 0.3 | 0.0 | 0.4 | 0.6 | 0.9 | 1.0 | 0.9 | 0.9 | 0.7 | 0.3 |
| 144 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.4 | 0.2 | 0.1 | 0.2 | 0.4 | 0.5 | 0.7 | 0.8 | 0.8 | 0.6 | 0.6 | 0.5 |
| 147 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.4 | 0.5 | 0.6 | 0.7 | 0.7 | 0.6 | 0.5 | 0.1 |
| 150 | 0.5 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.4 | 0.4 | 0.4 | 0.5 | 0.6 | 0.5 | 0.5 | 0.1 |
| 153 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.4 | 0.5 | 0.5 | 0.2 | 0.1 | 0.0 |
| 156 | 0.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.4 | 0.4 | 0.4 | 0.4 | 0.2 | 0.1 |
| 159 | 0.5 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.3 | 0.4 | 0.2 | 0.1 | 0.0 |
| 162 | 0.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.2 | 0.1 | 0.4 | 0.2 | 0.0 | 0.0 |
| 165 | 0.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| 168 | 1.3 | 0.5 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 171 | 1.6 | 0.6 | 0.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| 174 | 1.3 | 0.7 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 177 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| 180 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 2b. Luminous intensity values, azimuth 190-350°

Signature:

_________________________________

Print Name:

GH JOHN

_________________________________

Date:

________06-03-2013_________________________

Partner / Director
Duly authorised to sign on behalf of:

Photometric and Optical Testing Services LLP