| Photometric Test Report |  |  |
| --- | --- | --- |
| Report Number: POTS/GJ13057 | Report Date: 1402-2013 | Prepared By: G John |
| Test Laboratory: Photometric and Optical Testing Services, Cotswold Business Centre, 42 A P Ellis Road, Rissington Business Park, Upper Rissington, Gloucestershire, GL54 2QB |  |  |
| Company Registration Number: Registered in England & Wales No. OC352911 |  |  |
| Registered Address: Thistle Down Barn, Holcot Lane, Sywell, Northampton, NN6 0BG |  |  |

| Client Details |  |
| --- | --- |
| Company: Astro Lighting | Email: technical@astrolighting.co.uk |
| Address: Astro Lighting Limited, G2 River Way, Harlow CM20 2DP, Great Britain |  |

| Details of Product Tested |  |
| --- | --- |
| Manufacturer: Astro Lighting | Source Type: Domestic light |
| Model: TRIMLESS 230 V HALOGEN LAMP | Serial Number: 5624 |
| Lamp Type: HALOGEN |  |
| Power Supply Used: Uninterruptible AC power supply |  |
| Voltage(AC V) = 244.9 | Current (mA)= 276 |
| Power (Watts)= 67.52 | Power factor= 0.999 |

| Integrating Sphere Test |  |  |
| --- | --- | --- |
| Date of Test: 05-02-2013 |  | Ambient Temperature:25°C |
| Measurement Filename: |  |  |
| Instrument Used: Labsphere model CSLMS LED 4060 integrating sphere spectroradiometer |  |  |
| Integrating Sphere Size: 1m |  | Measurement Geometry (2 / 4): 4 |
| Sample Orientation: Horizontal |  | Auxiliary Correction Applied: YES |
| Comments: |  |  |
| Date of Last Calibration (Operating Hours): 08-11-2012 (02:36) |  | Spectral Flux Standard Lamp Used: SCL-1400 |
| Standard Lamp Serial Number: K75 |  | Traceable: to NIST standards |
| Calibration Certificate Number: DM-02008-001 |  | Calibration Certificate Date: 19th February 2010 |
| Calibration Lamp Uncertainty: ± 0.67% (k=2) |  |  |
| Results |  |  |
| Flux (lumens): 212.9 |  |  |
| CIE 1931 Chromaticity Cx: 0.4570 |  | CIE 1931 Chromaticity Cy: 0.4138 |
| CRI (%):98.49 |  | CCT (K): 2764 |

Figure 1: Spectral Irradiance

Figure 2: CIE 1931 diagram.

| Goniophotometer Test |  |  |  |
| --- | --- | --- | --- |
| Date of Test: 06-02-2013 |  | Ambient Temperature: 25°C |  |
| Measurement Filename: 5624 TRIMLESS 230 V HALOGEN LAMP |  |  |  |
| Instrument Used: Radiant Imaging NFMS0800 Goniometer with ProMetric PM-1423F-1 Imaging Photometer |  |  |  |
| Photometer Working Distance: 2 m |  | Measurement Geometry: Near-Field |  |
| Comments: |  |  |  |
| Reference Photometer Used: Specbos1201 |  | Reference Photometer Serial Number: 2911670 |  |
| Traceable: to NPL standards, UKAS Accredited |  | Calibration Certificate Number: 121104 |  |
| Calibration Certificate Date: 25th October 2011 |  | Sample Stabilisation Time (minutes):45 |  |
| Reference Photometer Calibration Uncertainty: ± 2.4% (k=2, 20-200 lux, CIE illuminant A source) |  |  |  |
| Scan Set Up |  |  |  |
| Direction | Range |  | Increment |
| Inclination Zone 1 | 0-90° |  | 3° |
| Azimuth | 0-360° |  | 10° |
| Results |  |  |  |
| Integrated Luminous Flux (lumens):212.9 | Peak Intensity (3° Spot, candelas): 412.3 |  | Efficacy (lumens/Watt): 3.15 |
| Beam Angle (50% of max intensity C0-180, degrees): 36.2 |  |  |  |
| Photometric Filename (IES LM-63-2002): 5624 TRIMLESS 230 V HALOGEN LAMP |  |  |  |
| IES File – Absolute or Relative Format? RELATIVE |  |  |  |
| Photometric Filename (EULUMDAT): 5624 TRIMLESS 230 V HALOGEN LAMP |  |  |  |
| EULUMDAT File – Absolute or Relative Format? RELATIVE |  |  |  |

Figure 3: Far-Field Luminous Intensity (C0-180, Cartesian Coordinates)

Figure 4: Far-Field Luminous Intensity (C0-180, C90-270, Polar Coordinates)

Figure 4. Illuminance cone diagram.

Table 1. UGR values

|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 |
| 3 | 408.9 | 411.9 | 412.2 | 411.6 | 412.3 | 411.4 | 409.2 | 408.9 | 406.6 | 403.6 | 399.3 | 399.2 | 394.8 | 393.9 | 390.6 | 386.7 | 387.5 | 387.5 | 383.2 |
| 6 | 375.1 | 380.8 | 379.4 | 381.7 | 382.4 | 381.9 | 380.4 | 377.9 | 376.1 | 370.6 | 365.9 | 359.2 | 354.9 | 350.2 | 345.2 | 341.7 | 338.1 | 341.0 | 336.9 |
| 9 | 323.5 | 331.3 | 336.3 | 339.3 | 343.6 | 345.2 | 344.7 | 342.1 | 340.2 | 334.8 | 328.0 | 318.6 | 312.4 | 305.0 | 298.2 | 290.7 | 287.7 | 286.3 | 284.0 |
| 12 | 273.4 | 283.5 | 293.9 | 303.4 | 307.7 | 312.9 | 317.8 | 318.0 | 316.1 | 311.6 | 304.5 | 295.2 | 285.4 | 273.3 | 262.9 | 256.5 | 249.9 | 245.8 | 243.6 |
| 15 | 233.2 | 248.2 | 262.3 | 279.0 | 288.1 | 296.6 | 303.9 | 306.2 | 305.5 | 302.6 | 298.6 | 292.6 | 281.1 | 269.8 | 259.1 | 249.4 | 240.9 | 232.4 | 223.2 |
| 18 | 202.1 | 219.3 | 239.5 | 257.3 | 276.7 | 291.4 | 301.7 | 305.4 | 306.2 | 307.5 | 310.2 | 308.2 | 301.8 | 286.9 | 272.6 | 258.6 | 243.7 | 226.9 | 209.3 |
| 21 | 174.8 | 191.9 | 213.1 | 236.1 | 260.1 | 283.1 | 297.4 | 304.2 | 308.5 | 315.3 | 324.0 | 326.5 | 318.2 | 300.6 | 278.2 | 257.4 | 234.4 | 209.7 | 187.7 |
| 24 | 144.8 | 159.1 | 177.6 | 202.9 | 230.1 | 254.9 | 270.7 | 281.2 | 287.9 | 298.4 | 305.6 | 310.4 | 300.5 | 282.1 | 257.5 | 231.0 | 200.9 | 175.7 | 156.2 |
| 27 | 110.3 | 120.2 | 137.3 | 155.6 | 173.8 | 194.9 | 212.2 | 223.7 | 230.5 | 239.4 | 242.0 | 247.2 | 239.0 | 222.1 | 202.2 | 182.7 | 158.1 | 134.0 | 119.9 |
| 30 | 74.9 | 80.6 | 87.4 | 102.7 | 111.2 | 123.6 | 138.0 | 141.4 | 153.4 | 158.5 | 161.5 | 159.1 | 161.3 | 148.9 | 137.4 | 127.4 | 106.5 | 94.2 | 84.6 |
| 33 | 45.0 | 46.0 | 50.3 | 54.9 | 62.5 | 68.4 | 71.4 | 79.9 | 86.2 | 91.6 | 93.9 | 94.6 | 89.9 | 88.8 | 81.9 | 71.9 | 66.1 | 58.8 | 55.2 |
| 36 | 25.1 | 24.2 | 27.5 | 30.8 | 33.3 | 36.3 | 39.0 | 44.1 | 46.5 | 50.2 | 51.9 | 53.4 | 49.9 | 47.4 | 43.0 | 39.3 | 37.3 | 34.1 | 34.1 |
| 39 | 14.4 | 14.0 | 14.8 | 17.2 | 18.2 | 19.7 | 21.1 | 22.8 | 25.7 | 26.0 | 28.1 | 26.6 | 25.8 | 23.7 | 22.1 | 20.6 | 19.3 | 20.2 | 20.1 |
| 42 | 9.2 | 9.0 | 9.4 | 10.0 | 10.0 | 10.6 | 11.3 | 11.8 | 12.8 | 12.7 | 13.3 | 13.1 | 12.9 | 11.6 | 11.3 | 11.3 | 11.3 | 11.6 | 11.6 |
| 45 | 6.5 | 6.3 | 6.4 | 6.6 | 6.4 | 6.4 | 6.4 | 6.5 | 6.8 | 6.8 | 6.9 | 7.1 | 7.3 | 6.9 | 7.0 | 7.0 | 6.9 | 7.1 | 7.3 |
| 48 | 4.7 | 4.4 | 4.6 | 4.5 | 4.4 | 4.2 | 3.7 | 4.0 | 4.3 | 4.2 | 4.4 | 4.5 | 4.5 | 4.6 | 4.7 | 4.6 | 4.6 | 5.0 | 5.0 |
| 51 | 3.4 | 3.1 | 3.5 | 3.3 | 3.2 | 2.9 | 2.5 | 2.9 | 3.1 | 2.7 | 3.0 | 3.1 | 3.1 | 3.1 | 3.4 | 3.4 | 3.4 | 3.6 | 3.6 |
| 54 | 2.5 | 2.3 | 2.6 | 2.6 | 2.4 | 2.2 | 1.8 | 2.0 | 2.1 | 1.8 | 2.0 | 2.1 | 2.2 | 2.2 | 2.5 | 2.5 | 2.3 | 2.6 | 2.5 |
| 57 | 2.0 | 1.8 | 2.0 | 2.0 | 1.9 | 1.6 | 1.4 | 1.4 | 1.4 | 1.2 | 1.4 | 1.4 | 0.7 | 1.5 | 1.7 | 1.7 | 1.5 | 1.8 | 1.8 |
| 60 | 1.6 | 1.5 | 1.7 | 1.7 | 1.5 | 1.3 | 1.2 | 0.7 | 0.9 | 0.7 | 1.0 | 0.6 | 1.0 | 1.2 | 0.9 | 1.1 | 1.1 | 1.3 | 1.3 |
| 63 | 1.4 | 1.3 | 1.5 | 1.4 | 1.3 | 1.1 | 1.0 | 0.5 | 0.7 | 0.5 | 0.8 | 0.7 | 0.4 | 0.5 | 1.0 | 0.8 | 0.4 | 1.0 | 1.0 |
| 66 | 1.3 | 1.3 | 1.4 | 1.3 | 1.2 | 1.0 | 0.0 | 0.6 | 0.6 | 0.2 | 0.7 | 0.3 | 0.3 | 0.8 | 0.8 | 0.3 | 0.7 | 0.8 | 0.7 |
| 69 | 1.2 | 1.2 | 1.2 | 1.2 | 1.1 | 0.9 | 0.8 | 0.4 | 0.3 | 0.1 | 0.6 | 0.5 | 0.3 | 0.8 | 0.8 | 0.0 | 0.4 | 0.7 | 0.6 |
| 72 | 1.2 | 1.1 | 1.1 | 1.1 | 1.0 | 0.9 | 0.4 | 0.4 | 0.2 | 0.1 | 0.0 | 0.3 | 0.0 | 0.7 | 0.3 | 0.3 | 0.4 | 0.3 | 0.4 |
| 75 | 0.9 | 1.0 | 1.1 | 1.1 | 1.0 | 0.4 | 0.7 | 0.3 | 0.0 | 0.0 | 0.4 | 0.0 | 0.5 | 0.6 | 0.6 | 0.0 | 0.0 | 0.0 | 0.4 |
| 78 | 1.0 | 0.5 | 0.8 | 0.5 | 0.5 | 0.8 | 0.3 | 0.1 | 0.2 | 0.1 | 0.2 | 0.2 | 0.0 | 0.4 | 0.4 | 0.3 | 0.4 | 0.0 | 0.2 |
| 81 | 0.7 | 0.4 | 0.9 | 0.9 | 0.9 | 0.4 | 0.3 | 0.2 | 0.0 | 0.4 | 0.0 | 0.1 | 0.2 | 0.5 | 0.3 | 0.2 | 0.0 | 0.0 | 0.2 |
| 84 | 0.5 | 0.6 | 0.9 | 0.9 | 0.8 | 0.8 | 0.3 | 0.0 | 0.0 | 0.1 | 0.2 | 0.0 | 0.2 | 0.3 | 0.3 | 0.0 | 0.2 | 0.0 | 0.3 |
| 87 | 0.7 | 0.3 | 0.6 | 0.8 | 0.8 | 0.5 | 0.3 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.2 | 0.2 | 0.0 | 0.1 | 0.0 | 0.0 |
| 90 | 0.4 | 0.5 | 0.3 | 0.4 | 0.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |

Table 2a. Luminous intensity values, azimuth 0-180°

|  | 190 | 200 | 210 | 220 | 230 | 240 | 250 | 260 | 270 | 280 | 290 | 300 | 310 | 320 | 330 | 340 | 350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 | 410.6 |
| 3 | 384.8 | 382.3 | 379.0 | 380.8 | 380.9 | 379.3 | 382.9 | 381.5 | 385.1 | 387.5 | 391.0 | 391.2 | 395.6 | 399.7 | 401.2 | 405.2 | 408.4 |
| 6 | 339.3 | 335.0 | 335.7 | 336.1 | 336.7 | 337.6 | 338.5 | 342.0 | 343.8 | 347.6 | 348.8 | 352.0 | 355.4 | 359.5 | 363.1 | 366.7 | 374.3 |
| 9 | 287.1 | 288.4 | 289.5 | 293.0 | 294.8 | 295.5 | 295.5 | 299.1 | 300.6 | 302.1 | 301.3 | 304.5 | 307.5 | 309.2 | 310.1 | 314.4 | 320.4 |
| 12 | 242.4 | 242.8 | 246.0 | 244.7 | 246.6 | 251.8 | 251.8 | 253.8 | 255.2 | 254.6 | 254.1 | 255.9 | 252.5 | 252.9 | 257.4 | 258.0 | 264.0 |
| 15 | 215.3 | 208.8 | 207.4 | 201.1 | 200.6 | 206.0 | 203.8 | 206.8 | 207.9 | 207.9 | 206.6 | 210.9 | 207.5 | 208.7 | 214.3 | 214.1 | 221.5 |
| 18 | 193.0 | 181.0 | 168.5 | 162.3 | 160.2 | 160.2 | 164.4 | 164.8 | 166.7 | 166.9 | 169.1 | 168.2 | 171.1 | 173.6 | 175.8 | 181.6 | 188.2 |
| 21 | 166.8 | 150.5 | 138.8 | 132.4 | 131.1 | 132.1 | 133.7 | 135.0 | 136.9 | 137.0 | 138.6 | 140.3 | 142.2 | 144.8 | 147.7 | 151.0 | 159.0 |
| 24 | 136.4 | 122.3 | 114.4 | 112.5 | 112.9 | 113.0 | 114.1 | 114.4 | 115.6 | 114.9 | 116.6 | 117.7 | 119.8 | 121.2 | 121.7 | 122.9 | 129.7 |
| 27 | 106.2 | 97.9 | 93.7 | 93.5 | 95.0 | 96.3 | 97.0 | 95.8 | 95.8 | 94.4 | 95.6 | 95.9 | 95.8 | 95.8 | 96.4 | 97.1 | 100.1 |
| 30 | 77.5 | 72.4 | 73.4 | 73.3 | 74.0 | 75.9 | 74.4 | 73.5 | 72.8 | 71.9 | 70.5 | 72.3 | 70.8 | 70.1 | 71.1 | 68.6 | 70.9 |
| 33 | 51.9 | 50.4 | 49.8 | 51.9 | 51.1 | 49.3 | 50.7 | 47.6 | 48.5 | 48.2 | 47.9 | 46.3 | 46.9 | 46.0 | 44.8 | 45.4 | 44.7 |
| 36 | 32.6 | 32.4 | 32.3 | 33.3 | 32.0 | 31.0 | 31.3 | 27.5 | 29.2 | 29.6 | 30.3 | 29.3 | 28.6 | 27.5 | 28.0 | 28.4 | 26.3 |
| 39 | 20.3 | 18.8 | 19.3 | 20.2 | 19.5 | 18.8 | 18.1 | 16.8 | 17.2 | 18.5 | 18.3 | 18.3 | 17.6 | 17.0 | 17.6 | 17.0 | 16.2 |
| 42 | 11.9 | 11.6 | 11.4 | 11.7 | 11.5 | 11.5 | 11.2 | 10.8 | 10.8 | 11.4 | 11.7 | 11.8 | 11.2 | 11.1 | 11.6 | 11.3 | 10.2 |
| 45 | 7.5 | 7.5 | 7.4 | 7.6 | 7.6 | 7.5 | 7.4 | 7.5 | 7.4 | 7.7 | 7.9 | 8.0 | 7.9 | 8.1 | 8.2 | 7.6 | 7.0 |
| 48 | 5.2 | 5.2 | 5.1 | 5.2 | 5.5 | 5.0 | 5.1 | 5.5 | 5.2 | 5.4 | 5.5 | 5.3 | 5.5 | 5.9 | 5.6 | 5.3 | 5.0 |
| 51 | 3.9 | 3.9 | 3.8 | 3.8 | 4.1 | 3.7 | 3.8 | 4.2 | 3.7 | 3.9 | 4.0 | 3.8 | 3.8 | 4.2 | 4.1 | 3.8 | 3.7 |
| 54 | 2.9 | 2.9 | 2.9 | 2.9 | 3.2 | 2.8 | 2.8 | 3.2 | 2.7 | 2.9 | 3.0 | 2.8 | 2.8 | 3.2 | 2.9 | 2.7 | 2.8 |
| 57 | 2.0 | 2.1 | 2.2 | 2.1 | 2.3 | 2.2 | 2.2 | 2.4 | 2.1 | 2.2 | 2.3 | 2.2 | 2.2 | 2.4 | 2.1 | 2.1 | 2.1 |
| 60 | 1.4 | 1.6 | 1.8 | 1.7 | 1.7 | 1.8 | 1.8 | 1.9 | 1.7 | 1.7 | 1.9 | 1.7 | 1.8 | 1.9 | 1.6 | 1.7 | 1.8 |
| 63 | 1.1 | 1.3 | 1.4 | 1.4 | 1.4 | 1.5 | 1.6 | 1.5 | 1.5 | 1.5 | 1.6 | 1.4 | 1.6 | 1.6 | 1.3 | 1.5 | 1.5 |
| 66 | 1.0 | 1.2 | 0.6 | 1.2 | 1.2 | 1.4 | 1.4 | 1.3 | 1.3 | 1.3 | 1.4 | 1.2 | 1.4 | 0.9 | 1.2 | 1.3 | 1.4 |
| 69 | 0.9 | 1.0 | 1.1 | 1.1 | 1.0 | 1.2 | 0.8 | 1.1 | 1.2 | 1.1 | 0.8 | 1.1 | 1.3 | 1.2 | 1.1 | 1.2 | 1.3 |
| 72 | 0.8 | 0.6 | 1.0 | 1.0 | 0.9 | 0.6 | 1.1 | 1.0 | 0.8 | 1.0 | 0.7 | 1.0 | 1.1 | 1.1 | 0.5 | 0.8 | 1.2 |
| 75 | 0.0 | 0.8 | 0.9 | 0.9 | 0.8 | 1.0 | 1.0 | 0.6 | 0.9 | 0.6 | 1.0 | 0.9 | 1.0 | 1.0 | 1.0 | 1.1 | 1.1 |
| 78 | 0.7 | 0.4 | 0.8 | 0.6 | 0.5 | 0.8 | 0.6 | 0.8 | 0.5 | 0.8 | 0.6 | 0.8 | 0.6 | 0.6 | 0.9 | 1.0 | 1.0 |
| 81 | 0.3 | 0.6 | 0.3 | 0.7 | 0.3 | 0.7 | 0.5 | 0.5 | 0.5 | 0.4 | 0.7 | 0.7 | 0.7 | 0.4 | 0.0 | 0.9 | 0.9 |
| 84 | 0.1 | 0.0 | 0.3 | 0.0 | 0.3 | 0.6 | 0.3 | 0.2 | 0.4 | 0.4 | 0.6 | 0.3 | 0.6 | 0.3 | 0.4 | 0.9 | 0.9 |
| 87 | 0.0 | 0.3 | 0.0 | 0.2 | 0.4 | 0.5 | 0.1 | 0.0 | 0.3 | 0.0 | 0.3 | 0.3 | 0.0 | 0.2 | 0.8 | 0.4 | 0.5 |
| 90 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.4 | 0.3 | 0.5 |

Table 2b. Luminous intensity values, azimuth 190-350°

Signature:

_________________________________

Print Name:

GH JOHN

_________________________________

Date:

14-02-2013

_________________________________

Partner / Director
Duly authorised to sign on behalf of:

Photometric and Optical Testing Services LLP