| Photometric Test Report |  |  |
| --- | --- | --- |
| Report Number: POTS/GJ12071 | Report Date: 18-12-2012 | Prepared By: G John |
| Test Laboratory: Photometric and Optical Testing Services, Cotswold Business Centre, 42 A P Ellis Road, Rissington Business Park, Upper Rissington, Gloucestershire, GL54 2QB |  |  |
| Company Registration Number: Registered in England & Wales No. OC352911 |  |  |
| Registered Address: Thistle Down Barn, Holcot Lane, Sywell, Northampton, NN6 0BG |  |  |

| Client Details |  |
| --- | --- |
| Company: Astro Lighting | Email: technical@astrolighting.co.uk |
| Address: Astro Lighting Limited, G2 River Way, Harlow CM20 2DP, Great Britain |  |

| Details of Product Tested |  |
| --- | --- |
| Manufacturer: Astro Lighting | Source Type: Domestic wall light |
| Model: PALERMO 900 HIGH OUTPUT | Serial Number: 0838 |
| Lamp Type: T5 |  |
| Power Supply Used: Uninterruptible AC power supply |  |
| Voltage(AC V) = 241.7 | Current (mA)= 143 |
| Power (Watts)= 37.3 | Power factor= 0.9866 |

| Integrating Sphere Test |  |  |
| --- | --- | --- |
| Date of Test: 3-12-2012 |  | Ambient Temperature:25°C |
| Measurement Filename: 0479 PALERMO 900 |  |  |
| Instrument Used: Labsphere model CSLMS LED 4060 integrating sphere spectroradiometer |  |  |
| Integrating Sphere Size: 1m |  | Measurement Geometry (2 / 4): 4 |
| Sample Orientation: Horizontal |  | Auxiliary Correction Applied: YES |
| Comments: |  |  |
| Date of Last Calibration (Operating Hours): 08-11-2012 (02:36) |  | Spectral Flux Standard Lamp Used: SCL-1400 |
| Standard Lamp Serial Number: K75 |  | Traceable: to NIST standards |
| Calibration Certificate Number: DM-02008-001 |  | Calibration Certificate Date: 19th February 2010 |
| Calibration Lamp Uncertainty: ± 0.67% (k=2) |  |  |
| Results |  |  |
| Flux (lumens):863.9 |  | Bare Lamp flux (lumens): 3200 |
| CIE 1931 Chromaticity Cx: 0.4046 |  | CIE 1931 Chromaticity Cy: 0.3795 |
| CRI (%): 82.95 |  | CCT (K): 3293 |
| LOR(%)= 27.0 |  |  |

Figure 1: Spectral Irradiance at 2 metres

Figure 2: CIE 1931 diagram.

| Goniophotometer Test |  |  |  |
| --- | --- | --- | --- |
| Date of Test: 3-12-2012 |  | Ambient Temperature: 25°C |  |
| Measurement Filename: 0838 PALERMO 900 HIGH OUTPUT |  |  |  |
| Instrument Used: Radiant Imaging NFMS0800 Goniometer with ProMetric PM-1423F-1 Imaging Photometer |  |  |  |
| Photometer Working Distance: 2m |  | Measurement Geometry: Near-Field |  |
| Comments: |  |  |  |
| Reference Photometer Used: Specbos1201 |  | Reference Photometer Serial Number: 2911670 |  |
| Traceable: to NPL standards, UKAS Accredited |  | Calibration Certificate Number: 121104 |  |
| Calibration Certificate Date: 25th October 2011 |  | Sample Stabilisation Time (minutes):26 |  |
| Reference Photometer Calibration Uncertainty: ± 2.4% (k=2, 20-200 lux, CIE illuminant A source) |  |  |  |
| Scan Set Up |  |  |  |
| Direction | Range |  | Increment |
| Inclination Zone 1 | 0-180° |  | 3° |
| Azimuth | 0-360° |  | 10° |
| Results |  |  |  |
| Integrated Luminous Flux (lumens): 863.9 | Peak Intensity (3° Spot, candelas): 128.7 |  | Efficacy (lumens/Watt): 23.16 |
| Beam Angle (50% of max intensity C0-180, degrees): 157.9 |  |  |  |
| Photometric Filename (IES LM-63-2002): 0838 PALERMO 900 HIGH OUTPUT |  |  |  |
| IES File – Absolute or Relative Format? Relative |  |  |  |
| Photometric Filename (EULUMDAT): 0838 PALERMO 900 HIGH OUTPUT |  |  |  |
| EULUMDAT File – Absolute or Relative Format? Relative |  |  |  |

Figure 3: Far-Field Luminous Intensity (C0-180, Cartesian Coordinates)

Figure 4: Far-Field Luminous Intensity (C0-180, C90-270, Polar Coordinates)

Figure 4. Illuminance cone diagram.

Table 1. UGR values

|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 |
| 3 | 126.6 | 126.0 | 125.0 | 125.5 | 125.8 | 126.6 | 126.8 | 126.0 | 124.7 | 124.4 | 125.2 | 127.6 | 129.2 | 128.7 | 125.0 | 122.3 | 120.4 | 121.5 | 123.1 |
| 6 | 123.1 | 124.4 | 125.2 | 123.6 | 124.4 | 125.0 | 127.4 | 130.3 | 130.3 | 128.4 | 128.4 | 130.0 | 129.5 | 126.3 | 123.6 | 123.1 | 123.4 | 123.4 | 125.5 |
| 9 | 117.2 | 122.0 | 125.0 | 121.5 | 123.6 | 125.8 | 128.2 | 126.3 | 125.0 | 130.3 | 132.7 | 129.5 | 127.6 | 126.6 | 123.6 | 123.1 | 123.9 | 121.5 | 123.9 |
| 12 | 117.2 | 124.4 | 120.4 | 121.8 | 125.2 | 125.0 | 126.3 | 127.4 | 128.4 | 129.5 | 132.7 | 131.6 | 127.9 | 125.2 | 124.2 | 126.0 | 122.6 | 119.9 | 125.5 |
| 15 | 118.0 | 118.5 | 119.6 | 122.0 | 122.6 | 127.6 | 127.6 | 127.9 | 128.7 | 133.8 | 131.1 | 130.0 | 127.9 | 126.8 | 129.5 | 122.3 | 124.7 | 123.1 | 124.7 |
| 18 | 115.9 | 119.3 | 118.0 | 123.4 | 128.4 | 125.2 | 127.9 | 130.8 | 131.6 | 130.8 | 132.7 | 130.6 | 132.7 | 126.3 | 127.4 | 120.7 | 118.0 | 115.6 | 119.6 |
| 21 | 115.3 | 116.4 | 113.7 | 119.3 | 119.6 | 129.0 | 131.1 | 130.0 | 138.3 | 132.7 | 134.0 | 131.6 | 132.4 | 132.2 | 126.3 | 121.5 | 116.7 | 113.2 | 119.6 |
| 24 | 111.9 | 111.6 | 114.3 | 116.4 | 123.9 | 123.9 | 129.2 | 131.9 | 135.6 | 136.4 | 132.2 | 133.8 | 129.8 | 127.4 | 122.8 | 119.6 | 115.1 | 113.2 | 114.3 |
| 27 | 106.0 | 112.1 | 110.3 | 111.9 | 117.5 | 128.7 | 127.4 | 136.4 | 135.6 | 139.1 | 138.3 | 133.2 | 130.6 | 125.8 | 119.9 | 118.5 | 113.5 | 110.8 | 112.4 |
| 30 | 106.0 | 105.7 | 107.9 | 116.4 | 119.3 | 119.6 | 129.8 | 133.0 | 138.6 | 139.6 | 136.4 | 137.0 | 132.7 | 129.2 | 123.6 | 120.2 | 111.1 | 109.2 | 110.3 |
| 33 | 102.8 | 101.7 | 107.1 | 114.0 | 117.5 | 127.1 | 131.1 | 130.3 | 133.2 | 141.5 | 139.1 | 135.9 | 129.2 | 127.1 | 118.8 | 115.9 | 110.8 | 107.9 | 107.1 |
| 36 | 96.9 | 98.3 | 101.5 | 110.8 | 120.2 | 122.6 | 127.6 | 133.0 | 137.5 | 134.8 | 137.8 | 136.4 | 131.1 | 125.2 | 119.6 | 116.9 | 102.8 | 103.3 | 101.7 |
| 39 | 95.6 | 93.2 | 101.5 | 109.5 | 114.0 | 124.2 | 129.2 | 133.0 | 137.5 | 134.6 | 140.2 | 138.3 | 129.2 | 124.2 | 119.6 | 110.8 | 101.5 | 96.4 | 99.6 |
| 42 | 88.6 | 87.6 | 99.6 | 103.6 | 113.5 | 122.3 | 131.1 | 134.0 | 134.3 | 136.2 | 137.8 | 136.4 | 131.1 | 122.8 | 120.4 | 107.6 | 95.9 | 94.3 | 91.8 |
| 45 | 85.7 | 81.7 | 92.4 | 99.9 | 114.8 | 115.9 | 128.4 | 134.3 | 139.1 | 132.2 | 134.8 | 133.8 | 127.9 | 121.5 | 115.9 | 102.3 | 95.9 | 91.0 | 90.2 |
| 48 | 79.0 | 80.6 | 87.6 | 98.8 | 109.2 | 118.3 | 125.2 | 130.0 | 134.8 | 139.1 | 136.2 | 135.6 | 129.5 | 120.4 | 110.0 | 100.7 | 87.8 | 85.2 | 83.8 |
| 51 | 73.4 | 73.2 | 87.3 | 95.3 | 104.1 | 115.6 | 124.7 | 127.4 | 131.4 | 137.8 | 140.2 | 134.3 | 128.7 | 123.6 | 111.9 | 96.1 | 86.2 | 79.6 | 73.4 |
| 54 | 65.9 | 67.6 | 78.0 | 95.3 | 99.3 | 111.1 | 120.2 | 129.8 | 134.3 | 134.8 | 135.6 | 134.0 | 126.0 | 115.9 | 104.9 | 92.4 | 86.0 | 74.2 | 72.6 |
| 57 | 57.7 | 61.1 | 73.2 | 90.8 | 99.6 | 110.5 | 117.5 | 125.0 | 130.0 | 132.2 | 135.9 | 128.2 | 122.8 | 115.3 | 101.2 | 86.5 | 78.8 | 69.2 | 62.5 |
| 60 | 56.9 | 56.6 | 68.9 | 82.2 | 94.5 | 112.9 | 116.1 | 125.8 | 131.1 | 131.9 | 137.0 | 129.5 | 118.8 | 112.7 | 103.3 | 86.8 | 75.3 | 63.0 | 56.6 |
| 63 | 48.9 | 55.3 | 61.4 | 75.6 | 90.0 | 105.2 | 119.3 | 126.6 | 127.9 | 132.7 | 130.3 | 127.1 | 119.9 | 110.5 | 96.1 | 83.0 | 70.0 | 56.9 | 49.4 |
| 66 | 39.2 | 44.1 | 54.7 | 74.8 | 93.2 | 99.3 | 111.9 | 122.3 | 128.2 | 131.1 | 130.0 | 126.0 | 115.3 | 105.2 | 91.3 | 81.4 | 63.5 | 49.1 | 45.7 |
| 69 | 34.2 | 38.2 | 52.1 | 71.0 | 87.8 | 101.7 | 110.5 | 121.5 | 124.7 | 125.8 | 125.0 | 121.0 | 112.1 | 100.7 | 89.7 | 75.8 | 57.9 | 46.2 | 36.8 |
| 72 | 28.0 | 36.6 | 49.7 | 65.7 | 86.2 | 95.9 | 110.0 | 116.9 | 123.9 | 123.1 | 122.8 | 117.7 | 112.1 | 102.0 | 86.5 | 70.5 | 55.5 | 39.2 | 28.6 |
| 75 | 20.3 | 28.3 | 45.4 | 61.4 | 78.0 | 92.6 | 108.1 | 113.7 | 123.4 | 127.4 | 121.0 | 117.5 | 111.9 | 100.7 | 82.5 | 68.9 | 50.5 | 32.6 | 23.5 |
| 78 | 14.4 | 22.2 | 40.3 | 61.1 | 74.5 | 91.3 | 102.0 | 112.9 | 120.4 | 121.8 | 120.4 | 113.5 | 108.7 | 98.5 | 78.8 | 65.4 | 44.1 | 27.8 | 14.7 |
| 81 | 9.1 | 18.4 | 38.4 | 53.1 | 70.0 | 88.1 | 98.3 | 106.5 | 114.8 | 119.1 | 118.8 | 114.8 | 106.8 | 96.4 | 77.2 | 61.7 | 41.9 | 21.9 | 10.1 |
| 84 | 4.5 | 15.0 | 33.4 | 53.1 | 68.6 | 82.2 | 92.6 | 105.2 | 112.9 | 117.5 | 114.8 | 107.6 | 101.7 | 90.2 | 74.2 | 59.8 | 38.7 | 18.4 | 5.6 |
| 87 | 2.4 | 11.2 | 28.8 | 47.5 | 65.1 | 78.5 | 91.8 | 102.8 | 106.5 | 108.7 | 106.8 | 105.5 | 96.1 | 84.1 | 74.2 | 53.4 | 32.8 | 15.0 | 2.4 |
| 90 | 0.0 | 8.5 | 24.8 | 41.1 | 61.7 | 76.1 | 84.6 | 100.4 | 99.1 | 112.4 | 107.3 | 99.6 | 89.2 | 79.3 | 63.8 | 48.9 | 31.5 | 12.5 | 0.0 |
| 93 | 0.0 | 8.5 | 24.8 | 41.1 | 61.7 | 76.1 | 84.6 | 100.4 | 99.1 | 112.4 | 107.3 | 99.6 | 89.2 | 79.3 | 63.8 | 48.9 | 31.5 | 12.5 | 0.0 |
| 96 | 0.5 | 8.5 | 25.4 | 39.8 | 54.7 | 67.8 | 75.6 | 90.0 | 102.3 | 107.1 | 102.0 | 92.4 | 78.0 | 69.2 | 58.2 | 40.3 | 28.3 | 10.9 | 1.1 |
| 99 | 0.5 | 5.9 | 20.3 | 33.6 | 45.1 | 55.3 | 72.1 | 84.6 | 97.2 | 102.8 | 95.6 | 87.0 | 73.2 | 57.7 | 44.6 | 35.0 | 22.7 | 7.2 | 0.5 |
| 102 | 0.0 | 3.7 | 17.4 | 31.2 | 44.6 | 56.3 | 73.2 | 80.6 | 94.3 | 98.5 | 93.2 | 82.2 | 73.2 | 55.8 | 43.8 | 30.7 | 18.7 | 5.3 | 0.0 |
| 105 | 0.5 | 4.8 | 17.4 | 30.2 | 46.2 | 57.7 | 73.2 | 81.2 | 92.6 | 93.7 | 92.4 | 83.0 | 72.6 | 58.7 | 45.7 | 31.2 | 18.7 | 4.8 | 0.5 |
| 108 | 0.5 | 4.3 | 15.8 | 31.2 | 44.3 | 56.3 | 69.2 | 78.8 | 87.6 | 90.0 | 88.4 | 79.3 | 68.4 | 57.7 | 45.1 | 30.7 | 17.4 | 6.1 | 0.5 |
| 111 | 1.1 | 4.3 | 15.0 | 28.3 | 42.2 | 54.7 | 67.8 | 79.3 | 82.8 | 88.4 | 84.6 | 77.4 | 65.4 | 54.7 | 43.3 | 30.7 | 18.2 | 7.2 | 1.1 |
| 114 | 0.5 | 4.3 | 14.4 | 27.8 | 39.8 | 53.4 | 66.2 | 74.0 | 80.6 | 81.7 | 78.2 | 73.2 | 62.5 | 51.5 | 40.3 | 27.5 | 16.3 | 7.7 | 1.1 |
| 117 | 0.0 | 4.8 | 13.9 | 25.1 | 39.5 | 51.5 | 64.9 | 70.8 | 75.0 | 78.2 | 77.4 | 72.6 | 61.4 | 48.6 | 37.4 | 25.9 | 15.5 | 7.2 | 1.1 |
| 120 | 0.5 | 4.3 | 13.9 | 23.5 | 37.1 | 46.7 | 60.1 | 66.8 | 70.8 | 72.6 | 69.7 | 67.3 | 60.6 | 47.0 | 34.7 | 25.4 | 13.9 | 7.2 | 1.1 |
| 123 | 0.5 | 3.7 | 13.1 | 23.0 | 34.2 | 46.7 | 56.3 | 64.3 | 67.3 | 67.8 | 67.8 | 64.3 | 55.3 | 44.6 | 34.7 | 24.6 | 14.4 | 6.1 | 1.1 |
| 126 | 0.5 | 3.7 | 11.5 | 22.7 | 33.6 | 43.3 | 53.9 | 59.0 | 61.9 | 63.5 | 61.1 | 58.7 | 53.4 | 44.3 | 32.3 | 21.6 | 13.9 | 6.1 | 1.1 |
| 129 | 0.5 | 3.7 | 10.9 | 21.1 | 31.2 | 39.8 | 48.1 | 54.7 | 58.2 | 58.7 | 57.7 | 55.8 | 50.5 | 40.9 | 30.7 | 21.6 | 12.5 | 5.9 | 1.1 |
| 132 | 0.5 | 2.9 | 10.1 | 19.2 | 27.0 | 37.9 | 45.7 | 49.9 | 52.9 | 56.3 | 54.2 | 49.9 | 45.1 | 38.4 | 28.3 | 19.8 | 12.0 | 5.3 | 1.3 |
| 135 | 0.5 | 2.9 | 9.1 | 16.8 | 25.4 | 35.0 | 42.2 | 46.7 | 49.4 | 52.3 | 49.9 | 47.0 | 42.7 | 34.7 | 27.0 | 18.7 | 10.9 | 5.3 | 1.3 |
| 138 | 0.5 | 2.9 | 8.5 | 16.3 | 24.0 | 30.2 | 37.1 | 42.2 | 46.7 | 46.2 | 45.1 | 43.3 | 37.9 | 33.1 | 25.4 | 16.8 | 9.6 | 4.8 | 1.3 |
| 141 | 0.5 | 2.4 | 8.5 | 15.0 | 21.6 | 29.9 | 34.2 | 38.4 | 42.7 | 42.7 | 42.7 | 39.5 | 34.7 | 30.7 | 23.5 | 16.3 | 9.1 | 4.3 | 1.9 |
| 144 | 0.5 | 2.4 | 6.7 | 13.9 | 20.6 | 24.6 | 31.2 | 35.0 | 37.4 | 37.9 | 38.4 | 36.0 | 31.2 | 25.9 | 21.1 | 14.4 | 7.7 | 3.7 | 1.3 |
| 147 | 0.5 | 2.4 | 6.7 | 11.5 | 17.4 | 23.0 | 27.8 | 31.2 | 34.2 | 33.6 | 33.6 | 32.3 | 28.3 | 23.5 | 19.2 | 13.4 | 7.2 | 3.7 | 1.3 |
| 150 | 0.5 | 1.9 | 5.9 | 10.7 | 15.5 | 21.1 | 26.4 | 27.8 | 29.9 | 31.2 | 29.9 | 28.8 | 25.4 | 21.6 | 16.8 | 12.0 | 6.7 | 2.9 | 1.3 |
| 153 | 0.5 | 1.3 | 4.8 | 8.5 | 13.9 | 16.8 | 22.2 | 24.6 | 25.9 | 26.4 | 27.5 | 25.1 | 22.2 | 18.7 | 13.4 | 10.1 | 6.1 | 2.9 | 1.1 |
| 156 | 0.0 | 1.3 | 4.3 | 7.7 | 10.9 | 15.8 | 18.7 | 20.6 | 23.0 | 24.0 | 23.0 | 21.6 | 19.2 | 16.3 | 12.5 | 8.5 | 4.8 | 2.4 | 1.1 |
| 159 | 0.5 | 1.3 | 3.5 | 6.1 | 9.1 | 11.5 | 15.8 | 16.3 | 19.2 | 18.7 | 19.2 | 18.7 | 15.5 | 13.1 | 10.1 | 6.7 | 4.3 | 2.4 | 1.1 |
| 162 | 0.5 | 1.1 | 2.9 | 4.8 | 7.2 | 10.1 | 12.5 | 13.9 | 15.0 | 15.8 | 16.3 | 14.4 | 12.0 | 11.5 | 8.5 | 6.1 | 3.5 | 1.9 | 1.1 |
| 165 | 0.5 | 1.1 | 1.9 | 3.5 | 6.1 | 8.3 | 9.6 | 10.9 | 12.0 | 12.0 | 12.0 | 10.9 | 10.1 | 8.3 | 6.1 | 4.8 | 2.9 | 1.3 | 1.1 |
| 168 | 0.5 | 0.5 | 1.3 | 3.5 | 4.8 | 5.9 | 6.7 | 8.5 | 8.5 | 9.1 | 8.5 | 8.5 | 7.2 | 6.1 | 5.3 | 3.5 | 2.4 | 1.3 | 1.1 |
| 171 | 0.5 | 0.5 | 1.3 | 2.4 | 3.5 | 4.3 | 4.8 | 5.3 | 6.1 | 6.1 | 6.1 | 6.1 | 5.3 | 4.3 | 3.5 | 1.9 | 1.3 | 1.1 | 0.5 |
| 174 | 0.5 | 0.5 | 1.1 | 1.9 | 1.9 | 2.4 | 3.5 | 3.5 | 3.7 | 3.7 | 3.7 | 3.7 | 3.5 | 2.9 | 2.4 | 1.9 | 1.3 | 0.5 | 0.5 |
| 177 | 0.0 | 0.5 | 1.1 | 1.1 | 1.1 | 1.3 | 1.3 | 1.9 | 2.4 | 2.4 | 2.4 | 2.4 | 1.9 | 1.3 | 1.3 | 1.3 | 1.1 | 0.5 | 0.5 |
| 180 | 0.5 | 0.0 | 0.5 | 0.0 | 0.5 | 1.1 | 1.1 | 1.1 | 1.1 | 0.5 | 0.5 | 1.1 | 1.1 | 1.1 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |

Table 2a. Luminous intensity values, azimuth 0-180°

|  | 190 | 200 | 210 | 220 | 230 | 240 | 250 | 260 | 270 | 280 | 290 | 300 | 310 | 320 | 330 | 340 | 350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 | 128.7 |
| 3 | 125.0 | 126.3 | 127.1 | 128.2 | 127.9 | 126.6 | 125.8 | 125.5 | 126.3 | 127.1 | 127.4 | 127.4 | 128.2 | 128.4 | 128.2 | 126.8 | 126.8 |
| 6 | 124.2 | 123.6 | 127.1 | 130.6 | 130.0 | 126.8 | 123.9 | 124.4 | 126.0 | 125.8 | 126.0 | 125.0 | 123.9 | 124.4 | 125.5 | 125.8 | 124.2 |
| 9 | 120.7 | 122.8 | 123.4 | 123.1 | 127.4 | 129.0 | 126.8 | 127.1 | 129.2 | 130.3 | 127.1 | 126.6 | 128.7 | 126.8 | 124.2 | 123.4 | 120.4 |
| 12 | 121.2 | 123.4 | 125.0 | 125.0 | 128.7 | 130.8 | 129.5 | 133.0 | 130.8 | 129.2 | 128.2 | 129.2 | 130.6 | 126.8 | 121.0 | 123.6 | 121.2 |
| 15 | 115.9 | 122.0 | 122.0 | 126.0 | 125.5 | 132.4 | 131.4 | 134.3 | 130.8 | 129.2 | 129.0 | 133.2 | 127.4 | 124.7 | 126.3 | 120.4 | 120.4 |
| 18 | 116.9 | 119.9 | 122.3 | 126.0 | 128.2 | 130.8 | 131.9 | 133.5 | 134.0 | 132.4 | 135.9 | 129.8 | 129.2 | 126.3 | 119.3 | 122.0 | 119.1 |
| 21 | 114.0 | 116.1 | 120.7 | 125.5 | 128.7 | 130.3 | 130.6 | 133.8 | 136.2 | 138.3 | 134.6 | 129.8 | 127.9 | 122.0 | 121.8 | 115.1 | 121.5 |
| 24 | 113.2 | 115.9 | 118.0 | 122.8 | 134.0 | 130.0 | 138.6 | 136.4 | 137.8 | 133.5 | 134.6 | 131.6 | 125.8 | 122.8 | 119.3 | 119.6 | 112.7 |
| 27 | 112.1 | 114.5 | 121.0 | 122.6 | 129.8 | 133.8 | 136.4 | 138.8 | 133.8 | 135.9 | 138.6 | 133.5 | 130.3 | 125.5 | 118.0 | 116.4 | 112.4 |
| 30 | 107.3 | 114.3 | 117.5 | 121.8 | 130.0 | 132.7 | 135.9 | 136.4 | 142.6 | 139.9 | 135.9 | 134.3 | 127.9 | 122.8 | 118.3 | 107.9 | 108.4 |
| 33 | 103.1 | 110.5 | 116.7 | 120.4 | 127.1 | 130.8 | 136.7 | 143.4 | 142.8 | 139.6 | 135.1 | 133.0 | 129.8 | 117.2 | 112.9 | 106.0 | 103.3 |
| 36 | 104.7 | 102.8 | 114.8 | 119.6 | 124.2 | 132.2 | 136.2 | 136.4 | 142.8 | 138.0 | 135.6 | 130.0 | 127.4 | 123.9 | 113.5 | 104.7 | 101.7 |
| 39 | 97.5 | 101.5 | 113.7 | 121.5 | 122.3 | 129.5 | 134.8 | 140.7 | 145.0 | 139.4 | 138.0 | 132.4 | 126.6 | 118.5 | 111.6 | 106.0 | 94.0 |
| 42 | 97.2 | 96.7 | 108.1 | 120.2 | 122.6 | 127.6 | 137.2 | 141.2 | 137.8 | 139.1 | 139.9 | 128.2 | 125.0 | 117.2 | 108.4 | 99.6 | 89.4 |
| 45 | 88.4 | 94.5 | 103.6 | 115.6 | 122.6 | 128.7 | 131.1 | 139.1 | 139.9 | 139.9 | 135.6 | 131.4 | 123.6 | 114.0 | 107.9 | 96.1 | 87.3 |
| 48 | 85.2 | 89.4 | 97.7 | 111.9 | 125.2 | 127.6 | 131.6 | 140.2 | 141.2 | 140.7 | 135.6 | 131.1 | 121.8 | 118.0 | 102.3 | 89.4 | 79.8 |
| 51 | 79.0 | 85.4 | 94.8 | 107.3 | 117.5 | 131.4 | 132.4 | 136.2 | 136.2 | 137.8 | 133.8 | 125.5 | 122.8 | 110.8 | 95.3 | 88.6 | 78.5 |
| 54 | 74.2 | 81.2 | 90.5 | 104.4 | 118.0 | 124.4 | 133.2 | 137.2 | 140.2 | 138.3 | 135.4 | 126.6 | 117.7 | 107.9 | 99.1 | 87.0 | 66.5 |
| 57 | 66.8 | 76.9 | 87.3 | 100.9 | 118.3 | 123.4 | 129.8 | 137.2 | 138.3 | 135.4 | 135.4 | 123.4 | 113.5 | 103.9 | 92.6 | 79.0 | 65.7 |
| 60 | 61.9 | 71.6 | 80.9 | 96.7 | 110.5 | 123.1 | 129.5 | 134.6 | 139.1 | 134.3 | 133.0 | 121.8 | 116.7 | 100.7 | 88.4 | 72.4 | 59.8 |
| 63 | 56.6 | 67.3 | 82.2 | 94.8 | 106.5 | 121.2 | 129.2 | 131.9 | 134.6 | 135.1 | 133.0 | 123.1 | 110.3 | 93.5 | 82.2 | 66.2 | 56.1 |
| 66 | 48.6 | 60.9 | 76.9 | 91.3 | 102.8 | 114.8 | 122.3 | 133.5 | 131.4 | 128.4 | 128.7 | 118.3 | 108.9 | 97.2 | 80.4 | 62.5 | 48.3 |
| 69 | 44.1 | 57.7 | 72.9 | 88.1 | 102.8 | 115.1 | 123.1 | 128.7 | 132.2 | 131.4 | 125.2 | 117.5 | 105.7 | 93.5 | 76.6 | 58.5 | 40.9 |
| 72 | 36.8 | 53.1 | 71.8 | 87.6 | 96.7 | 113.2 | 119.9 | 127.4 | 130.8 | 128.2 | 125.2 | 116.7 | 103.9 | 89.4 | 71.8 | 55.5 | 38.2 |
| 75 | 31.5 | 47.5 | 65.4 | 85.2 | 99.9 | 110.5 | 116.4 | 124.4 | 130.0 | 129.0 | 121.0 | 109.5 | 100.9 | 82.0 | 65.4 | 49.7 | 31.5 |
| 78 | 24.0 | 41.7 | 61.7 | 80.1 | 99.3 | 107.3 | 118.3 | 122.0 | 122.8 | 127.1 | 115.3 | 110.3 | 92.1 | 78.0 | 61.1 | 45.4 | 25.6 |
| 81 | 19.5 | 39.8 | 60.9 | 73.2 | 92.9 | 107.1 | 110.5 | 120.4 | 121.8 | 123.6 | 116.4 | 102.5 | 92.1 | 75.8 | 60.9 | 42.2 | 21.9 |
| 84 | 16.3 | 35.5 | 55.5 | 74.5 | 90.5 | 99.9 | 111.9 | 118.3 | 121.2 | 116.4 | 114.8 | 100.7 | 90.0 | 72.9 | 56.6 | 37.4 | 17.6 |
| 87 | 12.8 | 31.8 | 52.9 | 71.0 | 86.0 | 95.6 | 104.9 | 112.7 | 115.6 | 111.3 | 109.2 | 97.2 | 86.0 | 68.6 | 52.9 | 35.0 | 14.2 |
| 90 | 10.1 | 29.4 | 47.5 | 67.3 | 80.4 | 92.9 | 101.2 | 103.9 | 107.6 | 101.7 | 104.1 | 92.4 | 78.8 | 66.8 | 49.1 | 33.4 | 12.0 |
| 93 | 10.1 | 29.4 | 47.5 | 67.3 | 80.4 | 92.9 | 101.2 | 103.9 | 107.6 | 101.7 | 104.1 | 92.4 | 78.8 | 66.8 | 49.1 | 33.4 | 12.0 |
| 96 | 6.7 | 23.5 | 39.0 | 54.7 | 69.7 | 86.0 | 95.1 | 103.9 | 112.4 | 110.0 | 98.5 | 92.4 | 76.4 | 58.2 | 44.6 | 28.8 | 9.1 |
| 99 | 2.4 | 17.4 | 33.6 | 50.5 | 68.6 | 82.8 | 94.8 | 101.5 | 110.0 | 109.2 | 96.7 | 88.4 | 72.6 | 54.2 | 37.1 | 22.2 | 5.3 |
| 102 | 2.4 | 13.1 | 28.3 | 45.1 | 63.5 | 78.2 | 90.8 | 95.6 | 104.7 | 103.3 | 93.2 | 84.1 | 68.4 | 49.1 | 34.2 | 18.2 | 4.3 |
| 105 | 2.9 | 12.5 | 27.5 | 44.6 | 60.1 | 73.2 | 86.0 | 91.8 | 100.4 | 99.6 | 87.8 | 79.3 | 63.8 | 49.1 | 34.7 | 18.7 | 4.8 |
| 108 | 3.7 | 12.0 | 25.9 | 39.5 | 55.8 | 67.3 | 79.8 | 87.6 | 95.1 | 95.1 | 85.4 | 76.4 | 61.4 | 47.0 | 32.3 | 16.3 | 5.3 |
| 111 | 4.8 | 13.1 | 24.6 | 38.4 | 52.9 | 64.3 | 75.6 | 86.5 | 91.8 | 87.8 | 81.2 | 72.6 | 61.1 | 45.1 | 30.2 | 16.8 | 5.3 |
| 114 | 4.3 | 13.4 | 24.0 | 36.0 | 49.9 | 61.9 | 72.6 | 82.8 | 84.1 | 84.6 | 76.9 | 69.2 | 57.7 | 42.7 | 27.8 | 15.8 | 5.3 |
| 117 | 4.3 | 12.0 | 22.7 | 33.6 | 44.6 | 57.7 | 70.2 | 79.8 | 80.4 | 80.6 | 74.0 | 66.8 | 54.2 | 41.4 | 27.0 | 15.5 | 4.8 |
| 120 | 4.8 | 10.9 | 20.6 | 31.8 | 43.3 | 56.6 | 67.8 | 75.0 | 76.4 | 74.0 | 71.0 | 61.4 | 50.5 | 37.9 | 25.9 | 14.4 | 4.3 |
| 123 | 3.7 | 11.5 | 20.6 | 31.8 | 42.2 | 51.8 | 64.3 | 71.6 | 73.2 | 70.8 | 66.2 | 59.0 | 49.4 | 37.4 | 24.6 | 13.9 | 4.3 |
| 126 | 3.7 | 10.1 | 18.7 | 28.8 | 41.4 | 51.0 | 57.7 | 63.8 | 65.9 | 65.9 | 60.6 | 53.9 | 46.7 | 37.1 | 24.0 | 12.5 | 3.7 |
| 129 | 3.7 | 10.7 | 17.4 | 27.5 | 38.4 | 48.1 | 57.1 | 61.4 | 61.4 | 59.5 | 56.3 | 51.0 | 42.2 | 32.3 | 23.0 | 12.0 | 3.5 |
| 132 | 3.5 | 8.3 | 16.8 | 25.4 | 35.5 | 44.6 | 48.6 | 55.8 | 57.1 | 57.7 | 51.8 | 48.1 | 39.8 | 30.2 | 22.2 | 12.0 | 3.5 |
| 135 | 3.5 | 8.3 | 15.8 | 24.0 | 32.3 | 41.9 | 46.7 | 51.5 | 52.3 | 51.5 | 48.1 | 43.8 | 36.6 | 27.8 | 18.7 | 10.7 | 2.9 |
| 138 | 3.5 | 7.2 | 14.4 | 22.7 | 31.2 | 36.0 | 41.4 | 47.5 | 49.4 | 47.0 | 44.3 | 40.9 | 32.3 | 25.1 | 17.4 | 9.6 | 2.9 |
| 141 | 2.9 | 6.7 | 13.9 | 21.6 | 27.5 | 33.1 | 39.5 | 41.4 | 43.3 | 41.9 | 39.8 | 34.7 | 30.2 | 24.0 | 15.5 | 9.1 | 2.4 |
| 144 | 2.4 | 6.1 | 13.1 | 18.7 | 25.1 | 30.7 | 35.5 | 37.1 | 37.9 | 39.0 | 36.6 | 32.3 | 27.0 | 21.1 | 14.4 | 7.7 | 1.9 |
| 147 | 2.4 | 5.3 | 10.7 | 16.3 | 23.0 | 27.8 | 30.2 | 32.6 | 34.2 | 34.2 | 32.3 | 28.8 | 25.1 | 18.7 | 12.5 | 6.1 | 2.4 |
| 150 | 2.4 | 4.8 | 8.5 | 14.4 | 19.2 | 23.5 | 27.0 | 28.8 | 30.7 | 30.7 | 28.8 | 25.4 | 22.2 | 15.8 | 10.7 | 5.9 | 1.9 |
| 153 | 1.3 | 3.7 | 6.7 | 12.5 | 17.9 | 20.3 | 23.5 | 26.4 | 27.5 | 25.4 | 25.9 | 21.6 | 17.4 | 14.4 | 9.1 | 4.8 | 1.9 |
| 156 | 1.9 | 3.5 | 5.9 | 10.7 | 15.0 | 16.8 | 21.1 | 22.7 | 23.5 | 22.7 | 20.6 | 18.7 | 15.8 | 10.9 | 7.2 | 3.7 | 1.3 |
| 159 | 1.3 | 2.9 | 4.8 | 8.5 | 11.5 | 14.4 | 16.8 | 19.8 | 18.7 | 18.7 | 17.4 | 15.0 | 12.5 | 9.1 | 5.9 | 3.5 | 1.3 |
| 162 | 1.1 | 2.9 | 3.5 | 6.1 | 9.6 | 10.9 | 13.1 | 14.4 | 16.3 | 14.4 | 13.9 | 12.0 | 9.6 | 6.7 | 4.8 | 2.9 | 1.1 |
| 165 | 1.1 | 1.9 | 2.9 | 4.8 | 6.7 | 7.7 | 9.6 | 10.1 | 10.9 | 10.7 | 9.6 | 8.5 | 6.7 | 6.1 | 3.7 | 2.4 | 1.1 |
| 168 | 1.1 | 1.3 | 2.4 | 3.7 | 5.3 | 5.3 | 7.2 | 7.2 | 7.7 | 7.2 | 6.7 | 6.7 | 5.3 | 4.3 | 2.9 | 1.9 | 0.5 |
| 171 | 0.5 | 1.3 | 1.9 | 2.9 | 3.7 | 4.3 | 5.3 | 5.3 | 5.9 | 4.8 | 5.3 | 4.3 | 3.7 | 2.9 | 1.9 | 1.3 | 0.5 |
| 174 | 0.5 | 1.1 | 1.3 | 1.3 | 2.4 | 2.9 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 2.4 | 2.4 | 1.3 | 1.1 | 0.5 |
| 177 | 0.5 | 0.5 | 1.1 | 1.3 | 1.3 | 1.9 | 1.9 | 1.9 | 2.4 | 1.9 | 1.9 | 1.3 | 1.3 | 1.1 | 0.5 | 1.1 | 0.5 |
| 180 | 0.5 | 0.5 | 0.5 | 0.0 | 0.5 | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 | 0.5 | 0.5 | 0.5 | 0.5 |

Table 2b. Luminous intensity values, azimuth 190-350°

Signature:

_________________________________

Print Name:

GH JOHN

_________________________________

Date:

18-12-2012

_________________________________

Partner / Director
Duly authorised to sign on behalf of:

Photometric and Optical Testing Services LLP