| Photometric Test Report |  |  |
| --- | --- | --- |
| Report Number: POTS/GJ12074 | Report Date: 18-12-2012 | Prepared By: G John |
| Test Laboratory: Photometric and Optical Testing Services, Cotswold Business Centre, 42 A P Ellis Road, Rissington Business Park, Upper Rissington, Gloucestershire, GL54 2QB |  |  |
| Company Registration Number: Registered in England & Wales No. OC352911 |  |  |
| Registered Address: Thistle Down Barn, Holcot Lane, Sywell, Northampton, NN6 0BG |  |  |

| Client Details |  |
| --- | --- |
| Company: Astro Lighting | Email: technical@astrolighting.co.uk |
| Address: Astro Lighting Limited, G2 River Way, Harlow CM20 2DP, Great Britain |  |

| Details of Product Tested |  |
| --- | --- |
| Manufacturer: Astro Lighting | Source Type: Domestic wall light |
| Model: PALERMO 1200 | Serial Number: 0627 |
| Lamp Type: T5 |  |
| Power Supply Used: Uninterruptible AC power supply |  |
| Voltage(AC V) = 241.7 | Current (mA)= 143 |
| Power (Watts)= 34.07 | Power factor= 0.9866 |

| Integrating Sphere Test |  |  |
| --- | --- | --- |
| Date of Test: 5-12-2012 |  | Ambient Temperature:25°C |
| Measurement Filename: 7037 ROMANO 900 HIGH OUTPUT |  |  |
| Instrument Used: Labsphere model CSLMS LED 4060 integrating sphere spectroradiometer |  |  |
| Integrating Sphere Size: 1m |  | Measurement Geometry (2 / 4): 4 |
| Sample Orientation: Horizontal |  | Auxiliary Correction Applied: YES |
| Comments: |  |  |
| Date of Last Calibration (Operating Hours): 08-11-2012 (02:36) |  | Spectral Flux Standard Lamp Used: SCL-1400 |
| Standard Lamp Serial Number: K75 |  | Traceable: to NIST standards |
| Calibration Certificate Number: DM-02008-001 |  | Calibration Certificate Date: 19th February 2010 |
| Calibration Lamp Uncertainty: ± 0.67% (k=2) |  |  |
| Results |  |  |
| Flux (lumens): 1014.2 |  | Bare Lamp flux (lumens): 3200 |
| CIE 1931 Chromaticity Cx: 0.4410 |  | CIE 1931 Chromaticity Cy: 0.4086 |
| CRI (%): 82.27 |  | CCT (K): 2967 |
| LOR(%)= 31.6 |  |  |

Figure 1: Spectral Irradiance at 2 metres

Figure 2: CIE 1931 diagram.

| Goniophotometer Test |  |  |  |
| --- | --- | --- | --- |
| Date of Test: 5-12-2012 |  | Ambient Temperature: 25°C |  |
| Measurement Filename: 7037 ROMANO 900 HIGH OUTPUT |  |  |  |
| Instrument Used: Radiant Imaging NFMS0800 Goniometer with ProMetric PM-1423F-1 Imaging Photometer |  |  |  |
| Photometer Working Distance: 23m |  | Measurement Geometry: Near-Field |  |
| Comments: |  |  |  |
| Reference Photometer Used: Specbos1201 |  | Reference Photometer Serial Number: 2911670 |  |
| Traceable: to NPL standards, UKAS Accredited |  | Calibration Certificate Number: 121104 |  |
| Calibration Certificate Date: 25th October 2011 |  | Sample Stabilisation Time (minutes):26 |  |
| Reference Photometer Calibration Uncertainty: ± 2.4% (k=2, 20-200 lux, CIE illuminant A source) |  |  |  |
| Scan Set Up |  |  |  |
| Direction | Range |  | Increment |
| Inclination Zone 1 | 0-180° |  | 3° |
| Azimuth | 0-360° |  | 10° |
| Results |  |  |  |
| Integrated Luminous Flux (lumens): 1014.2 | Peak Intensity (3° Spot, candelas): 173.3 |  | Efficacy (lumens/Watt): 29.76 |
| Beam Angle (50% of max intensity C0-180, degrees): 141.7 |  |  |  |
| Photometric Filename (IES LM-63-2002): 7037 ROMANO 900 HIGH OUTPUT |  |  |  |
| IES File – Absolute or Relative Format? Relative |  |  |  |
| Photometric Filename (EULUMDAT): 7037 ROMANO 900 HIGH OUTPUT |  |  |  |
| EULUMDAT File – Absolute or Relative Format? Relative |  |  |  |

Figure 3: Far-Field Luminous Intensity (C0-180, Cartesian Coordinates)

Figure 4: Far-Field Luminous Intensity (C0-180, C90-270, Polar Coordinates)

Figure 5. Illuminance cone diagram.

Table 1. UGR values

|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 |
| 3 | 138.3 | 138.1 | 137.0 | 136.8 | 137.4 | 137.9 | 137.6 | 137.2 | 137.9 | 138.7 | 137.7 | 136.8 | 135.0 | 133.9 | 133.7 | 134.0 | 135.5 | 137.2 | 137.7 |
| 6 | 130.8 | 133.6 | 138.4 | 136.5 | 133.6 | 132.9 | 132.6 | 137.0 | 143.0 | 143.0 | 140.9 | 141.3 | 141.6 | 140.9 | 137.4 | 135.2 | 135.0 | 135.9 | 139.2 |
| 9 | 129.7 | 136.6 | 140.8 | 133.2 | 135.5 | 137.3 | 140.6 | 138.3 | 137.3 | 143.7 | 146.6 | 144.1 | 139.8 | 140.8 | 141.3 | 137.0 | 136.8 | 131.0 | 133.2 |
| 12 | 130.7 | 132.2 | 134.1 | 135.9 | 141.6 | 144.5 | 143.2 | 145.3 | 147.9 | 146.7 | 149.3 | 145.2 | 148.8 | 145.7 | 138.6 | 139.0 | 133.9 | 126.8 | 136.5 |
| 15 | 126.7 | 132.2 | 133.7 | 138.7 | 141.3 | 142.0 | 147.1 | 146.1 | 152.5 | 149.9 | 152.5 | 147.8 | 147.7 | 143.4 | 137.6 | 137.7 | 133.7 | 127.7 | 137.9 |
| 18 | 123.1 | 131.5 | 135.0 | 134.0 | 141.9 | 142.3 | 145.9 | 154.6 | 158.6 | 154.6 | 152.2 | 154.8 | 149.5 | 148.5 | 143.5 | 136.1 | 135.5 | 127.1 | 136.6 |
| 21 | 120.3 | 129.7 | 125.0 | 142.3 | 141.7 | 149.3 | 151.1 | 152.9 | 158.4 | 162.2 | 157.3 | 155.7 | 146.6 | 147.2 | 146.1 | 134.8 | 134.8 | 121.0 | 134.6 |
| 24 | 122.5 | 124.1 | 125.9 | 134.1 | 141.3 | 146.4 | 154.4 | 158.1 | 158.6 | 164.4 | 157.7 | 158.7 | 154.0 | 148.6 | 144.5 | 132.9 | 132.3 | 120.1 | 127.8 |
| 27 | 115.6 | 121.6 | 127.0 | 131.0 | 138.6 | 154.7 | 154.1 | 163.4 | 160.9 | 164.2 | 165.3 | 162.6 | 153.6 | 153.9 | 140.5 | 135.5 | 133.9 | 116.6 | 127.8 |
| 30 | 120.6 | 117.3 | 127.1 | 137.2 | 144.6 | 150.1 | 162.0 | 159.3 | 164.9 | 167.1 | 162.4 | 163.5 | 154.1 | 152.6 | 140.1 | 136.9 | 126.5 | 115.8 | 125.6 |
| 33 | 113.9 | 113.7 | 122.5 | 133.4 | 142.3 | 151.4 | 156.6 | 159.9 | 163.8 | 164.2 | 170.7 | 164.4 | 156.6 | 154.0 | 139.4 | 143.5 | 120.6 | 114.0 | 118.7 |
| 36 | 103.6 | 108.6 | 118.0 | 129.6 | 147.5 | 148.1 | 159.4 | 164.2 | 166.6 | 168.8 | 168.2 | 162.4 | 158.3 | 148.4 | 137.6 | 135.4 | 119.4 | 113.6 | 114.7 |
| 39 | 98.1 | 101.3 | 114.1 | 132.5 | 134.7 | 153.9 | 157.3 | 163.4 | 167.0 | 169.5 | 166.6 | 163.1 | 156.6 | 146.7 | 139.7 | 132.3 | 112.3 | 109.3 | 108.6 |
| 42 | 94.9 | 99.8 | 112.3 | 120.8 | 141.2 | 154.3 | 152.8 | 171.7 | 167.5 | 173.3 | 165.3 | 164.4 | 153.6 | 147.4 | 139.4 | 128.5 | 110.7 | 107.1 | 104.3 |
| 45 | 91.8 | 88.9 | 103.6 | 120.5 | 134.0 | 144.5 | 153.6 | 164.4 | 166.2 | 166.4 | 167.8 | 166.7 | 158.0 | 142.7 | 146.6 | 124.1 | 100.9 | 99.4 | 100.6 |
| 48 | 86.4 | 87.4 | 96.5 | 115.2 | 132.6 | 141.3 | 153.6 | 159.9 | 168.6 | 166.2 | 169.7 | 161.6 | 150.1 | 146.1 | 138.0 | 120.9 | 101.0 | 95.9 | 95.4 |
| 51 | 80.7 | 82.5 | 102.0 | 108.6 | 131.1 | 138.1 | 148.8 | 156.1 | 170.2 | 165.0 | 163.1 | 160.6 | 146.0 | 139.2 | 134.4 | 110.0 | 95.9 | 90.9 | 87.4 |
| 54 | 69.8 | 78.0 | 94.9 | 110.0 | 120.1 | 136.8 | 152.8 | 163.3 | 165.7 | 160.9 | 158.6 | 155.4 | 148.5 | 141.9 | 122.8 | 108.3 | 94.3 | 85.6 | 79.8 |
| 57 | 66.1 | 71.8 | 93.6 | 105.0 | 117.6 | 131.2 | 142.7 | 155.8 | 162.3 | 157.5 | 158.4 | 150.8 | 142.7 | 137.2 | 122.5 | 101.6 | 87.1 | 78.5 | 72.2 |
| 60 | 62.7 | 66.0 | 77.7 | 96.5 | 114.5 | 135.0 | 145.6 | 153.5 | 155.4 | 161.5 | 158.1 | 148.2 | 137.9 | 133.2 | 116.9 | 98.0 | 84.7 | 71.8 | 65.7 |
| 63 | 57.7 | 62.0 | 76.7 | 90.7 | 107.4 | 125.4 | 139.1 | 148.8 | 152.2 | 150.6 | 149.7 | 147.1 | 139.9 | 125.7 | 108.1 | 96.6 | 77.8 | 65.8 | 61.4 |
| 66 | 45.4 | 54.2 | 70.5 | 93.0 | 106.8 | 120.1 | 131.0 | 146.8 | 147.9 | 149.5 | 149.7 | 144.8 | 135.8 | 123.4 | 102.4 | 89.0 | 73.7 | 61.3 | 53.3 |
| 69 | 38.0 | 46.6 | 64.0 | 85.4 | 102.0 | 117.4 | 125.3 | 138.0 | 147.7 | 148.2 | 146.0 | 139.8 | 130.1 | 121.7 | 98.8 | 80.2 | 67.6 | 53.0 | 43.5 |
| 72 | 34.1 | 42.6 | 57.5 | 79.6 | 95.1 | 112.2 | 123.9 | 130.5 | 138.1 | 136.6 | 140.5 | 134.8 | 124.1 | 108.7 | 97.6 | 77.4 | 58.9 | 47.1 | 36.7 |
| 75 | 25.5 | 36.6 | 52.7 | 68.6 | 85.0 | 100.3 | 120.2 | 128.2 | 140.2 | 137.2 | 139.7 | 129.2 | 113.9 | 102.5 | 91.5 | 75.3 | 54.6 | 37.4 | 27.5 |
| 78 | 18.4 | 31.3 | 50.0 | 63.2 | 80.2 | 97.8 | 114.1 | 126.4 | 135.0 | 135.9 | 128.8 | 127.4 | 110.5 | 102.9 | 81.1 | 64.7 | 48.7 | 30.9 | 19.5 |
| 81 | 11.7 | 24.0 | 41.0 | 57.1 | 77.6 | 91.1 | 108.7 | 121.2 | 127.0 | 125.4 | 130.0 | 120.1 | 109.2 | 93.2 | 80.0 | 60.6 | 42.6 | 22.2 | 11.9 |
| 84 | 5.9 | 18.4 | 36.4 | 52.6 | 70.9 | 88.0 | 108.2 | 116.7 | 123.8 | 124.5 | 125.2 | 113.4 | 102.8 | 86.3 | 70.9 | 54.1 | 37.5 | 17.8 | 6.3 |
| 87 | 2.2 | 13.9 | 33.1 | 50.6 | 61.8 | 83.6 | 104.1 | 114.8 | 119.6 | 122.1 | 121.0 | 109.0 | 101.6 | 84.3 | 68.4 | 54.0 | 34.1 | 14.8 | 3.2 |
| 90 | 1.5 | 10.6 | 32.3 | 49.8 | 68.4 | 81.6 | 91.2 | 102.8 | 110.5 | 111.5 | 113.6 | 100.5 | 94.4 | 81.3 | 70.7 | 50.1 | 32.2 | 14.6 | 1.2 |
| 93 | 1.4 | 10.6 | 30.4 | 50.1 | 73.0 | 82.9 | 95.1 | 103.4 | 110.8 | 112.5 | 106.4 | 101.7 | 91.2 | 84.3 | 71.5 | 49.3 | 29.4 | 14.2 | 2.1 |
| 96 | 1.4 | 9.8 | 30.6 | 50.1 | 70.0 | 80.2 | 94.0 | 102.5 | 108.6 | 116.1 | 106.4 | 102.1 | 94.7 | 86.3 | 70.1 | 51.2 | 32.0 | 12.3 | 1.7 |
| 99 | 1.4 | 9.7 | 28.3 | 48.9 | 66.8 | 80.9 | 94.7 | 102.0 | 108.1 | 115.6 | 110.0 | 101.8 | 96.2 | 81.6 | 68.2 | 49.1 | 30.2 | 12.7 | 2.5 |
| 102 | 2.1 | 8.3 | 24.0 | 44.9 | 62.4 | 78.8 | 92.5 | 100.9 | 108.3 | 112.2 | 109.3 | 101.3 | 92.5 | 78.9 | 63.1 | 46.4 | 27.2 | 13.7 | 2.5 |
| 105 | 1.0 | 9.2 | 22.1 | 40.6 | 62.0 | 78.5 | 88.6 | 99.9 | 106.4 | 109.0 | 109.0 | 98.4 | 86.8 | 78.9 | 65.7 | 44.9 | 27.7 | 13.5 | 2.2 |
| 108 | 1.1 | 10.2 | 27.5 | 43.6 | 58.0 | 73.8 | 86.3 | 97.6 | 103.6 | 106.7 | 107.9 | 96.3 | 81.6 | 72.7 | 62.4 | 44.6 | 25.9 | 13.7 | 2.1 |
| 111 | 1.4 | 9.1 | 26.5 | 42.6 | 54.2 | 70.5 | 88.2 | 90.5 | 98.1 | 103.9 | 103.9 | 93.4 | 84.2 | 71.6 | 59.9 | 42.8 | 25.4 | 12.7 | 3.2 |
| 114 | 1.4 | 8.8 | 25.5 | 38.5 | 56.3 | 68.2 | 78.5 | 93.4 | 100.6 | 98.4 | 97.8 | 94.3 | 82.9 | 69.6 | 57.7 | 43.5 | 25.4 | 14.1 | 3.0 |
| 117 | 1.4 | 8.6 | 21.8 | 38.8 | 54.1 | 64.6 | 74.4 | 89.1 | 94.4 | 93.6 | 89.6 | 88.6 | 78.8 | 66.9 | 52.6 | 41.5 | 24.4 | 12.7 | 3.5 |
| 120 | 1.5 | 8.8 | 21.0 | 39.2 | 52.6 | 61.0 | 72.2 | 82.0 | 94.4 | 93.6 | 87.6 | 88.2 | 77.7 | 62.8 | 54.4 | 40.8 | 22.4 | 12.1 | 3.7 |
| 123 | 1.5 | 8.6 | 21.3 | 35.3 | 47.5 | 63.5 | 70.0 | 80.2 | 88.2 | 89.4 | 86.4 | 80.7 | 76.9 | 61.1 | 49.3 | 38.0 | 22.4 | 12.6 | 4.0 |
| 126 | 1.1 | 8.3 | 20.7 | 31.2 | 46.4 | 62.7 | 67.1 | 75.1 | 82.1 | 85.7 | 83.5 | 74.0 | 71.8 | 60.3 | 45.4 | 37.3 | 22.4 | 11.0 | 3.0 |
| 129 | 1.4 | 7.3 | 19.2 | 32.0 | 45.5 | 53.7 | 67.3 | 69.7 | 75.9 | 82.4 | 79.2 | 72.3 | 69.1 | 58.4 | 43.6 | 33.9 | 20.7 | 11.7 | 3.9 |
| 132 | 1.5 | 7.5 | 16.7 | 30.9 | 38.5 | 52.3 | 61.0 | 64.4 | 70.5 | 74.5 | 74.9 | 69.7 | 63.1 | 53.1 | 41.0 | 32.8 | 19.6 | 10.1 | 3.2 |
| 135 | 1.7 | 6.8 | 16.6 | 28.2 | 37.5 | 51.8 | 54.1 | 63.2 | 67.2 | 68.3 | 68.4 | 66.8 | 55.2 | 51.1 | 38.2 | 31.5 | 19.3 | 10.1 | 2.5 |
| 138 | 1.8 | 5.4 | 14.8 | 26.1 | 37.0 | 44.6 | 53.4 | 59.3 | 63.5 | 65.6 | 60.2 | 60.9 | 51.5 | 48.2 | 37.0 | 28.2 | 17.0 | 9.8 | 2.6 |
| 141 | 1.8 | 5.5 | 14.5 | 22.1 | 33.9 | 41.8 | 47.6 | 54.1 | 58.7 | 60.7 | 58.5 | 56.2 | 48.7 | 42.8 | 34.4 | 25.1 | 16.6 | 8.1 | 2.1 |
| 144 | 1.8 | 4.7 | 11.7 | 21.9 | 29.5 | 36.6 | 44.7 | 49.7 | 52.9 | 52.7 | 52.6 | 48.6 | 47.1 | 39.5 | 31.5 | 22.6 | 14.8 | 8.1 | 2.6 |
| 147 | 1.7 | 4.6 | 11.5 | 19.0 | 25.9 | 33.8 | 41.3 | 42.5 | 46.9 | 49.1 | 49.5 | 45.8 | 42.0 | 34.4 | 28.4 | 19.9 | 13.7 | 6.9 | 2.5 |
| 150 | 1.7 | 4.1 | 9.4 | 16.0 | 24.7 | 29.5 | 35.2 | 38.9 | 41.1 | 45.0 | 43.5 | 41.3 | 34.9 | 29.1 | 25.1 | 17.1 | 12.6 | 6.2 | 1.8 |
| 153 | 1.1 | 4.0 | 8.6 | 14.2 | 20.8 | 25.1 | 31.1 | 35.5 | 37.1 | 37.3 | 36.6 | 35.2 | 29.5 | 27.3 | 22.8 | 15.0 | 11.2 | 6.8 | 2.9 |
| 156 | 1.7 | 3.3 | 7.6 | 12.7 | 17.1 | 22.2 | 26.6 | 29.5 | 31.1 | 32.2 | 32.7 | 29.8 | 27.0 | 24.4 | 18.4 | 13.2 | 8.6 | 6.5 | 2.2 |
| 159 | 1.5 | 2.9 | 6.2 | 9.7 | 14.4 | 18.8 | 23.0 | 24.0 | 24.6 | 25.8 | 25.1 | 23.3 | 22.4 | 19.5 | 14.5 | 10.5 | 8.0 | 4.8 | 1.8 |
| 162 | 1.4 | 2.5 | 5.1 | 8.1 | 11.5 | 14.4 | 15.7 | 19.3 | 20.6 | 19.9 | 20.0 | 20.6 | 16.8 | 14.4 | 11.5 | 9.4 | 6.6 | 4.7 | 2.1 |
| 165 | 1.8 | 2.5 | 3.9 | 5.7 | 8.6 | 10.1 | 11.9 | 14.4 | 15.0 | 16.4 | 16.0 | 14.5 | 12.8 | 11.5 | 9.9 | 7.3 | 5.9 | 4.1 | 1.9 |
| 168 | 1.4 | 2.3 | 3.2 | 4.3 | 5.7 | 6.8 | 7.7 | 9.1 | 10.4 | 10.5 | 10.6 | 9.9 | 8.0 | 7.6 | 7.2 | 5.7 | 4.3 | 2.9 | 2.1 |
| 171 | 1.1 | 2.1 | 2.6 | 3.2 | 4.0 | 4.8 | 5.8 | 5.9 | 5.7 | 6.3 | 6.6 | 6.1 | 5.7 | 5.5 | 4.8 | 4.3 | 3.3 | 2.3 | 1.5 |
| 174 | 1.5 | 2.2 | 2.5 | 2.8 | 2.9 | 3.2 | 3.5 | 3.6 | 3.7 | 3.7 | 3.7 | 3.5 | 3.5 | 3.2 | 2.9 | 2.6 | 1.1 | 2.1 | 1.4 |
| 177 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 |
| 180 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 2a. Luminous intensity values, azimuth 0-180°

|  | 190 | 200 | 210 | 220 | 230 | 240 | 250 | 260 | 270 | 280 | 290 | 300 | 310 | 320 | 330 | 340 | 350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 | 140.3 |
| 3 | 138.3 | 136.6 | 134.6 | 135.2 | 136.8 | 138.1 | 137.2 | 136.6 | 134.0 | 133.7 | 133.9 | 137.4 | 138.8 | 139.7 | 138.7 | 138.6 | 139.0 |
| 6 | 138.6 | 135.7 | 136.1 | 137.4 | 137.0 | 135.4 | 138.1 | 139.0 | 138.1 | 141.0 | 143.0 | 139.0 | 133.9 | 136.1 | 137.4 | 134.7 | 132.2 |
| 9 | 132.2 | 136.1 | 133.0 | 139.1 | 140.2 | 137.7 | 140.1 | 142.0 | 143.9 | 140.8 | 143.8 | 143.8 | 140.8 | 138.6 | 136.1 | 137.2 | 133.3 |
| 12 | 131.2 | 136.2 | 138.8 | 137.9 | 143.8 | 146.6 | 144.1 | 143.9 | 144.3 | 153.3 | 148.6 | 141.9 | 142.0 | 138.1 | 139.0 | 133.7 | 134.3 |
| 15 | 129.0 | 139.0 | 138.1 | 136.8 | 145.0 | 144.3 | 145.9 | 155.9 | 151.9 | 148.9 | 149.5 | 150.6 | 142.6 | 143.1 | 136.6 | 131.4 | 135.8 |
| 18 | 123.1 | 136.9 | 135.2 | 141.9 | 146.8 | 148.6 | 153.0 | 155.7 | 151.4 | 159.3 | 158.3 | 152.2 | 146.0 | 144.6 | 135.1 | 131.1 | 133.6 |
| 21 | 125.6 | 137.3 | 131.5 | 146.7 | 147.4 | 155.3 | 162.4 | 160.6 | 154.6 | 159.1 | 157.0 | 155.3 | 152.4 | 140.3 | 139.2 | 129.4 | 128.5 |
| 24 | 119.0 | 134.3 | 132.6 | 146.8 | 144.3 | 157.0 | 158.7 | 164.2 | 162.3 | 159.1 | 162.4 | 158.8 | 149.9 | 143.4 | 137.7 | 132.8 | 124.2 |
| 27 | 116.7 | 130.4 | 139.0 | 144.8 | 151.0 | 157.6 | 163.0 | 169.3 | 166.6 | 162.4 | 164.1 | 157.6 | 153.3 | 138.3 | 133.9 | 126.3 | 123.9 |
| 30 | 115.4 | 126.8 | 136.9 | 142.6 | 157.9 | 158.8 | 166.6 | 171.3 | 167.8 | 166.0 | 164.6 | 157.7 | 149.9 | 144.3 | 127.7 | 127.2 | 116.1 |
| 33 | 110.8 | 124.3 | 132.8 | 139.7 | 155.8 | 160.6 | 163.3 | 171.0 | 171.4 | 169.6 | 165.9 | 156.4 | 153.0 | 140.8 | 132.9 | 121.6 | 113.0 |
| 36 | 109.8 | 122.1 | 137.9 | 138.7 | 153.7 | 162.0 | 165.3 | 177.2 | 170.4 | 173.6 | 165.0 | 162.0 | 146.0 | 144.8 | 129.4 | 117.2 | 106.4 |
| 39 | 110.3 | 115.2 | 132.5 | 144.5 | 149.2 | 157.7 | 165.5 | 171.5 | 172.1 | 170.2 | 166.3 | 156.1 | 152.1 | 134.7 | 128.2 | 112.1 | 100.2 |
| 42 | 105.7 | 112.5 | 130.8 | 142.1 | 149.5 | 159.8 | 169.6 | 171.5 | 171.4 | 173.7 | 167.5 | 154.0 | 147.9 | 137.4 | 119.5 | 116.7 | 98.5 |
| 45 | 101.3 | 105.3 | 127.5 | 142.0 | 153.0 | 160.8 | 166.0 | 172.5 | 174.8 | 168.8 | 160.4 | 155.9 | 145.6 | 133.7 | 121.3 | 104.2 | 89.3 |
| 48 | 95.6 | 99.1 | 121.3 | 136.3 | 147.8 | 155.8 | 163.4 | 173.1 | 174.3 | 170.8 | 162.2 | 153.7 | 146.8 | 133.0 | 112.9 | 96.0 | 83.6 |
| 51 | 86.7 | 98.0 | 119.8 | 134.0 | 144.5 | 154.4 | 165.0 | 170.3 | 171.9 | 170.7 | 161.5 | 155.5 | 141.5 | 128.1 | 109.2 | 101.7 | 80.3 |
| 54 | 84.3 | 90.8 | 113.3 | 133.9 | 148.5 | 153.7 | 159.4 | 165.5 | 175.5 | 168.4 | 164.2 | 153.3 | 133.4 | 122.8 | 107.8 | 91.8 | 74.7 |
| 57 | 81.1 | 91.2 | 107.2 | 126.0 | 141.5 | 148.5 | 161.5 | 161.5 | 171.3 | 166.7 | 161.5 | 144.1 | 131.1 | 115.2 | 101.0 | 87.1 | 65.8 |
| 60 | 73.8 | 86.1 | 101.2 | 123.9 | 137.0 | 155.8 | 159.7 | 161.2 | 164.1 | 162.6 | 158.0 | 140.3 | 131.9 | 115.9 | 97.2 | 79.2 | 63.9 |
| 63 | 67.1 | 77.1 | 95.6 | 115.4 | 137.4 | 149.3 | 149.5 | 158.8 | 163.3 | 158.7 | 149.0 | 139.9 | 129.7 | 107.0 | 89.4 | 71.3 | 61.1 |
| 66 | 64.0 | 74.5 | 93.2 | 113.0 | 126.5 | 143.5 | 146.7 | 152.5 | 158.3 | 158.3 | 146.4 | 136.8 | 119.5 | 102.7 | 89.7 | 71.1 | 52.4 |
| 69 | 56.2 | 67.9 | 87.2 | 111.9 | 123.4 | 133.0 | 144.9 | 155.8 | 151.9 | 150.7 | 140.9 | 130.0 | 119.5 | 101.2 | 82.7 | 62.1 | 44.0 |
| 72 | 47.9 | 63.8 | 83.2 | 100.9 | 117.3 | 133.4 | 145.6 | 145.3 | 148.5 | 144.2 | 137.6 | 127.0 | 108.7 | 94.7 | 78.2 | 60.0 | 40.8 |
| 75 | 36.2 | 57.1 | 77.6 | 97.8 | 111.1 | 127.5 | 139.2 | 141.6 | 145.0 | 143.9 | 132.1 | 115.8 | 103.6 | 89.0 | 71.5 | 56.2 | 34.5 |
| 78 | 31.9 | 51.8 | 70.4 | 91.2 | 106.7 | 119.4 | 135.7 | 135.7 | 139.1 | 137.9 | 130.3 | 114.7 | 104.9 | 80.6 | 66.7 | 49.4 | 28.7 |
| 81 | 23.2 | 46.1 | 65.1 | 82.5 | 101.3 | 113.4 | 128.8 | 136.1 | 137.6 | 131.4 | 128.9 | 111.8 | 96.0 | 78.9 | 58.7 | 41.8 | 21.9 |
| 84 | 20.1 | 41.4 | 62.1 | 76.3 | 97.6 | 111.4 | 123.5 | 130.1 | 132.8 | 133.2 | 117.3 | 108.5 | 87.4 | 76.3 | 54.4 | 38.5 | 17.4 |
| 87 | 14.6 | 35.2 | 55.6 | 71.3 | 95.1 | 111.8 | 118.1 | 122.4 | 124.3 | 124.3 | 114.1 | 97.3 | 83.6 | 70.4 | 50.5 | 32.6 | 12.1 |
| 90 | 10.1 | 27.2 | 51.5 | 74.5 | 89.8 | 99.8 | 110.5 | 118.1 | 125.2 | 120.8 | 116.3 | 108.3 | 94.0 | 79.5 | 58.8 | 34.6 | 13.2 |
| 93 | 8.7 | 26.8 | 48.0 | 75.1 | 90.0 | 100.6 | 108.9 | 117.6 | 121.0 | 120.5 | 116.2 | 108.2 | 93.2 | 73.6 | 53.4 | 32.3 | 10.9 |
| 96 | 8.3 | 25.4 | 49.8 | 70.1 | 85.7 | 94.8 | 102.5 | 117.3 | 117.2 | 118.3 | 110.0 | 101.8 | 87.6 | 71.9 | 54.1 | 31.6 | 9.7 |
| 99 | 7.3 | 25.0 | 47.2 | 66.0 | 82.9 | 94.7 | 103.2 | 113.2 | 120.5 | 116.7 | 107.2 | 98.9 | 84.2 | 67.8 | 50.1 | 28.7 | 8.7 |
| 102 | 9.1 | 25.8 | 45.7 | 65.4 | 78.5 | 90.9 | 99.9 | 109.2 | 119.5 | 119.2 | 106.5 | 97.3 | 82.8 | 63.8 | 46.2 | 25.4 | 8.4 |
| 105 | 8.4 | 21.8 | 43.5 | 63.1 | 75.3 | 85.3 | 102.9 | 109.4 | 113.9 | 108.5 | 105.0 | 95.5 | 77.4 | 65.0 | 42.5 | 25.9 | 9.0 |
| 108 | 10.6 | 20.7 | 40.7 | 59.2 | 73.4 | 81.1 | 96.7 | 107.1 | 110.0 | 105.8 | 99.6 | 93.2 | 75.5 | 60.0 | 42.4 | 26.1 | 8.0 |
| 111 | 10.1 | 19.9 | 39.9 | 56.7 | 69.1 | 83.1 | 96.5 | 103.9 | 103.1 | 102.3 | 98.8 | 91.8 | 74.7 | 55.8 | 43.2 | 24.4 | 7.7 |
| 114 | 9.5 | 20.8 | 38.4 | 53.0 | 65.1 | 81.4 | 92.9 | 95.9 | 97.3 | 99.2 | 95.5 | 84.3 | 72.7 | 57.5 | 40.2 | 23.2 | 7.9 |
| 117 | 9.4 | 18.8 | 38.9 | 51.6 | 63.3 | 78.4 | 93.4 | 94.0 | 96.0 | 96.3 | 93.3 | 81.7 | 65.7 | 54.4 | 40.7 | 21.1 | 7.3 |
| 120 | 8.7 | 17.1 | 35.1 | 49.4 | 58.9 | 77.0 | 87.4 | 89.0 | 94.4 | 93.8 | 84.2 | 77.1 | 65.1 | 52.2 | 36.8 | 20.1 | 7.2 |
| 123 | 7.5 | 17.9 | 34.4 | 45.1 | 61.4 | 73.0 | 82.5 | 83.5 | 90.1 | 90.1 | 83.5 | 73.8 | 62.7 | 48.7 | 35.7 | 19.6 | 6.5 |
| 126 | 7.2 | 18.5 | 30.9 | 42.8 | 56.9 | 71.3 | 77.0 | 82.1 | 83.5 | 80.7 | 75.5 | 68.2 | 60.4 | 47.3 | 30.8 | 17.9 | 7.2 |
| 129 | 6.9 | 15.6 | 30.1 | 39.2 | 54.4 | 66.7 | 70.5 | 80.2 | 81.0 | 77.6 | 73.7 | 63.9 | 55.1 | 45.5 | 31.6 | 18.4 | 6.5 |
| 132 | 6.6 | 15.0 | 26.4 | 38.0 | 51.3 | 58.7 | 64.0 | 74.4 | 73.0 | 73.0 | 67.1 | 61.0 | 51.8 | 38.6 | 30.4 | 17.1 | 5.7 |
| 135 | 5.8 | 13.5 | 24.3 | 34.8 | 47.1 | 52.4 | 60.3 | 65.8 | 68.7 | 69.7 | 62.0 | 55.8 | 49.5 | 37.1 | 26.6 | 15.6 | 5.5 |
| 138 | 5.8 | 13.2 | 23.6 | 31.7 | 43.1 | 49.0 | 57.5 | 58.5 | 64.3 | 60.3 | 59.1 | 53.7 | 45.7 | 37.4 | 25.7 | 13.2 | 5.2 |
| 141 | 5.1 | 13.4 | 20.4 | 29.8 | 38.6 | 46.0 | 51.8 | 55.6 | 58.5 | 58.7 | 51.5 | 49.0 | 38.5 | 31.5 | 22.4 | 11.9 | 5.4 |
| 144 | 4.4 | 11.0 | 18.4 | 28.0 | 35.1 | 41.5 | 45.5 | 51.6 | 52.2 | 52.9 | 48.6 | 43.5 | 36.8 | 29.3 | 19.9 | 11.7 | 5.0 |
| 147 | 3.6 | 9.4 | 15.6 | 25.7 | 29.4 | 37.3 | 41.4 | 46.2 | 47.6 | 44.4 | 43.2 | 41.0 | 33.7 | 25.1 | 19.6 | 10.1 | 3.9 |
| 150 | 3.7 | 8.8 | 13.9 | 21.7 | 27.2 | 33.7 | 37.7 | 38.8 | 42.9 | 41.3 | 37.0 | 33.5 | 28.6 | 22.2 | 16.0 | 9.4 | 4.3 |
| 153 | 2.8 | 7.2 | 11.5 | 17.7 | 24.2 | 26.9 | 32.3 | 33.4 | 36.6 | 34.6 | 36.7 | 31.3 | 25.0 | 18.4 | 13.8 | 8.0 | 3.5 |
| 156 | 2.3 | 6.1 | 9.7 | 14.4 | 19.5 | 23.7 | 26.5 | 30.6 | 29.5 | 27.3 | 26.8 | 24.3 | 21.4 | 16.7 | 10.6 | 6.6 | 3.0 |
| 159 | 2.5 | 5.4 | 8.6 | 11.6 | 16.7 | 19.0 | 21.8 | 23.6 | 24.3 | 24.7 | 22.8 | 20.0 | 16.1 | 13.4 | 9.0 | 5.0 | 2.3 |
| 162 | 2.3 | 4.1 | 6.9 | 9.0 | 11.0 | 14.4 | 16.3 | 17.9 | 18.6 | 18.1 | 16.3 | 15.6 | 14.2 | 10.4 | 7.2 | 4.1 | 2.2 |
| 165 | 2.2 | 3.3 | 4.7 | 6.8 | 8.3 | 9.4 | 11.7 | 13.1 | 13.8 | 14.2 | 12.1 | 10.2 | 10.1 | 6.9 | 5.0 | 3.0 | 2.1 |
| 168 | 1.8 | 3.0 | 3.9 | 4.7 | 6.2 | 7.2 | 7.7 | 8.6 | 8.4 | 8.6 | 7.9 | 6.8 | 5.8 | 4.8 | 3.6 | 2.6 | 2.1 |
| 171 | 1.7 | 1.9 | 2.2 | 2.6 | 3.7 | 4.3 | 4.4 | 4.7 | 5.0 | 4.8 | 4.7 | 4.1 | 3.3 | 2.6 | 2.2 | 1.9 | 1.9 |
| 174 | 1.5 | 1.5 | 1.8 | 1.9 | 2.3 | 2.5 | 2.5 | 2.6 | 2.5 | 2.5 | 2.2 | 2.2 | 1.9 | 1.8 | 1.7 | 1.7 | 1.0 |
| 177 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 |
| 180 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 2b. Luminous intensity values, azimuth 190-350°

Signature:

_________________________________

Print Name:

GH JOHN

_________________________________

Date:

18-12-2012

_________________________________

Partner / Director
Duly authorised to sign on behalf of:

Photometric and Optical Testing Services LLP